Solutions Manual

for

Introduction to Thermodynamics and Heat Transfer

Yunus A. Cengel

2nd Edition, 2008

Chapter 15

# RADIATION HEAT TRANSFER

# PROPRIETARY AND CONFIDENTIAL

This Manual is the proprietary property of The McGraw-Hill Companies, Inc. (“McGraw-Hill”) and protected by copyright and other state and federal laws.  By opening and using this Manual the user agrees to the following restrictions, and if the recipient does not agree to these restrictions, the Manual should be promptly returned unopened to McGraw-Hill:  This Manual is being provided only to authorized professors and instructors for use in preparing for the classes using the affiliated textbook.  No other use or distribution of this Manual is permitted.  This Manual may not be sold and may not be distributed to or used by any student or other third party.  No part of this Manual may be reproduced, displayed or distributed in any form or by any means, electronic or otherwise, without the prior written permission of McGraw-Hill.

Electromagnetic and Thermal Radiation

15-1C Electromagnetic waves are caused by accelerated charges or changing electric currents giving rise to electric and magnetic fields. Sound waves are caused by disturbances.  Electromagnetic waves can travel in vacuum, sound waves cannot.

15-2C Electromagnetic waves are characterized by their frequency v and wavelength λ.  These two properties in a medium are related by λ=cvwhere c is the speed of light in that medium.

15-3C Visible light is a kind of electromagnetic wave whose wavelength is between 0.40 and 0.76 m. It differs from the other forms of electromagnetic radiation in that it triggers the sensation of seeing in the human eye.

15-4C Infrared radiation lies between 0.76 and 100 m whereas ultraviolet radiation lies between the wavelengths 0.01 and 0.40 m.  The human body does not emit any radiation in the ultraviolet region since bodies at room temperature emit radiation in the infrared region only.

15-5C Thermal radiation is the radiation emitted as a result of vibrational and rotational motions of molecules, atoms and electrons of a substance, and it extends from about 0.1 to 100  in wavelength. Unlike the other forms of electromagnetic radiation, thermal radiation is emitted by bodies because of their temperature.

15-6C Light (or visible) radiation consists of narrow bands of colors from violet to red. The color of a surface depends on its ability to reflect certain wavelength. For example, a surface that reflects radiation in the wavelength range 0.63-0.76 m while absorbing the rest appears red to the eye. A surface that reflects all the light appears white while a surface that absorbs the entire light incident on it appears black. The color of a surface at room temperature is not related to the radiation it emits.

15-7C Radiation in opaque solids is considered surface phenomena since only radiation emitted by the molecules in a very thin layer of a body at the surface can escape the solid.

15-8C Because the snow reflects almost all of the visible and ultraviolet radiation, and the skin is exposed to radiation both from the sun and from the snow.

15-9C Microwaves in the range of 102 to 105μm are very suitable for use in cooking as they are reflected by metals, transmitted by glass and plastics and absorbed by food (especially water) molecules. Thus the electric energy converted to radiation in a microwave oven eventually becomes part of the internal energy of the food with no conduction and convection thermal resistances involved. In conventional cooking, on the other hand, conduction and convection thermal resistances slow down the heat transfer, and thus the heating process.

15-10 The speeds of light in air, water, and glass are to be determined.

Analysis The speeds of light in air, water and glass are

- Air:	c=c0n=3.0×108 m/s1=3.0×108 m/s

- Water:	c=c0n=3.0×108 m/s1.33=2.26×108 m/s

- Glass:	c=c0n=3.0×108 m/s1.5=2.0×108 m/s

15-11 Electricity is generated and transmitted in power lines at a frequency of 60 Hz. The wavelength of the electromagnetic waves is to be determined.

Power lines

Analysis The wavelength of the electromagnetic waves is

λ=cv=2.998×108 m/s60 Hz(1/s)=4.997×106 m

15-12 A microwave oven operates at a frequency of 2.2109 Hz. The wavelength of these microwaves and the energy of each microwave are to be determined.

Microwave oven

Analysis The wavelength of these microwaves is

λ=cv=2.998×108 m/s2.2×109 Hz(1/s)=0.136 m=136 mm

Then the energy of each microwave becomes

e=hv=hcλ=(6.625×10−34 Js)(2.998×108 m/s)0.136 m=1.46×10−24 J

15-13 A radio station is broadcasting radiowaves at a wavelength of 200 m. The frequency of these waves is to be determined.

Analysis The frequency of the waves is determined from

λ=cvv=cλ=2.998×108 m/s200 m =1.5×106 Hz

15-14 A cordless telephone operates at a frequency of 8.5108 Hz. The wavelength of these telephone waves is to be determined.

Analysis The wavelength of the telephone waves is

λ=cv=2.998×108 m/s8.5×108 Hz(1/s)=0.353 m=353 mm

# Blackbody Radiation

15-15C A blackbody is a perfect emitter and absorber of radiation.  A blackbody does not actually exist.  It is an idealized body that emits the maximum amount of radiation that can be emitted by a surface at a given temperature.

15-16C Spectral blackbody emissive power is the amount of radiation energy emitted by a blackbody at an absolute temperature T per unit time, per unit surface area and per unit wavelength about wavelength λ.  The integration of the spectral blackbody emissive power over the entire wavelength spectrum gives the total blackbody emissive power,

Eb(T)=0∞Ebλ(T)dλ=σT4

The spectral blackbody emissive power varies with wavelength, the total blackbody emissive power does not.

15-17C We defined the blackbody radiation function fλ because the integration 0∞Ebλ(T)dλcannot be performed. The blackbody radiation function fλ represents the fraction of radiation emitted from a blackbody at temperature T in the wavelength range from λ= 0 to λ. This function is used to determine the fraction of radiation in a wavelength range between λ1 and λ2.

15-18C The larger the temperature of a body, the larger the fraction of the radiation emitted in shorter wavelengths. Therefore, the body at 1500 K will emit more radiation in the shorter wavelength region. The body at 1000 K emits more radiation at  20 m than the body at 1500 K since λT=constant.

15-19 The maximum thermal radiation that can be emitted by a surface is to be determined.

Analysis The maximum thermal radiation that can be emitted by a surface is determined from Stefan-Boltzman law to be

Eb(T)=σT4=(5.67×10−8 W/m2.K4)(800 K)4=2.32×104 W/m2

15-20 An isothermal cubical body is suspended in the air. The rate at which the cube emits radiation energy and the spectral blackbody emissive power are to be determined.

Assumptions The body behaves as a black body.

Analysis (a) The total blackbody emissive power is determined from Stefan-Boltzman Law to be

As=6a2=6(0.22)=0.24 m2

T = 750 K

20 cm

20 cm

20 cm

Eb(T)=σT4As=(5.67×10−8 W/m2.K4)(750 K)4(0.24 m2)=4306 W

(b) The spectral blackbody emissive power at a wavelength of 4  is determined from Plank's distribution law,

Ebλ=C1λ5expC2λT−1=3.74177×108 W⋅μm4/m2(4μm)5exp1.43878×104μm⋅K(4μm)(750 K)−13045 W/m2⋅μm=3.05 kW/m2⋅μm

15-21E The sun is at an effective surface temperature of 10,400 R. The rate of infrared radiation energy emitted by the sun is to be determined.

Assumptions The sun behaves as a black body.

SUN

T = 10,400 R

Analysis Noting that T = 10,400 R = 5778 K, the blackbody radiation functions corresponding to λ1T and λ2T are determined from Table 15-2 to be

λ1T=(0.76μm)(5778 K)=4391.3 μmKfλ1=0.547370λ2T=(100μm)(5778 K)=577,800 μmKfλ2=1.0

Then the fraction of radiation emitted between these two wavelengths becomes

fλ2−fλ1=1.0−0.547=0.453     (or 45.3%)

The total blackbody emissive power of the sun is determined from Stefan-Boltzman Law to be

Eb=σT4=(0.1714×10−8 Btu/h.ft2.R4)(10,400 R)4=2.005×107 Btu/h.ft2

Then,

Einfrared=(0.453)Eb=(0.453)(2.005×107 Btu/h.ft2)=9.08×106 Btu/h.ft2

15-22 EES The spectral blackbody emissive power of the sun versus wavelength in the range of 0.01 m to 1000 m is to be plotted.

Analysis The problem is solved using EES, and the solution is given below.

"GIVEN"

T=5780 [K]

lambda=0.01[micrometer]

"ANALYSIS"

E_b_lambda=C_1/(lambda^5*(exp(C_2/(lambda*T))-1))

C_1=3.742E8 [W-micrometer^4/m^2]

C_2=1.439E4 [micrometer-K]

|  [m] | Eb,  [W/m2-m] |
| --- | --- |
| 0.01 | 0 |
| 10.11 | 12684 |
| 20.21 | 846.3 |
| 30.31 | 170.8 |
| 40.41 | 54.63 |
| 50.51 | 22.52 |
| 60.62 | 10.91 |
| 70.72 | 5.905 |
| 80.82 | 3.469 |
| 90.92 | 2.17 |
| … | … |
| … | … |
| 909.1 | 0.0002198 |
| 919.2 | 0.0002103 |
| 929.3 | 0.0002013 |
| 939.4 | 0.0001928 |
| 949.5 | 0.0001847 |
| 959.6 | 0.000177 |
| 969.7 | 0.0001698 |
| 979.8 | 0.0001629 |
| 989.9 | 0.0001563 |
| 1000 | 0.0001501 |

15-23 The temperature of the filament of an incandescent light bulb is given. The fraction of visible radiation emitted by the filament and the wavelength at which the emission peaks are to be determined.

Assumptions The filament behaves as a black body.

T = 3200 K

Analysis The visible range of the electromagnetic spectrum extends from λ1=0.40μm to λ2=0.76μm .  Noting that  T = 3200 K, the blackbody radiation functions corresponding to λ1T and λ2T are determined from Table 15-2 to be

λ1T=(0.40μm)(3200 K)=1280 μmKfλ1=0.0043964λ2T=(0.76μm)(3200 K)=2432 μmKfλ2=0.147114

Then the fraction of radiation emitted between these two wavelengths becomes

fλ2−fλ1=0.147114−0.004396=0.142718     (or 14.3%)

The wavelength at which the emission of radiation from the filament is maximum is

(λT)max power=2897.8μm⋅Kλmax power=2897.8μm⋅K3200 K=0.906 mm

15-24 EES Prob. 15-23 is reconsidered. The effect of temperature on the fraction of radiation emitted in the visible range is to be investigated.

Analysis The problem is solved using EES, and the solution is given below.

"GIVEN"

T=3200 [K]

lambda_1=0.40 [micrometer]

lambda_2=0.76 [micrometer]

"ANALYSIS"

E_b_lambda=C_1/(lambda^5*(exp(C_2/(lambda*T))-1))

C_1=3.742E8 [W-micrometer^4/m^2]

C_2=1.439E4 [micrometer-K]

f_lambda=integral(E_b_lambda, lambda, lambda_1, lambda_2)/E_b

E_b=sigma*T^4

sigma=5.67E-8 [W/m^2-K^4] “Stefan-Boltzmann constant"

| T [K] | f |
| --- | --- |
| 1000 | 0.000007353 |
| 1200 | 0.0001032 |
| 1400 | 0.0006403 |
| 1600 | 0.002405 |
| 1800 | 0.006505 |
| 2000 | 0.01404 |
| 2200 | 0.02576 |
| 2400 | 0.04198 |
| 2600 | 0.06248 |
| 2800 | 0.08671 |
| 3000 | 0.1139 |
| 3200 | 0.143 |
| 3400 | 0.1732 |
| 3600 | 0.2036 |
| 3800 | 0.2336 |
| 4000 | 0.2623 |

15-25 An incandescent light bulb emits 15% of its energy at wavelengths shorter than 0.8 m. The temperature of the filament is to be determined.

T = ?

Assumptions The filament behaves as a black body.

Analysis From the Table 15-2 for the fraction of the radiation, we read

fλ=0.15λT=2445μmK

For the wavelength range of λ1=0.0μm to λ2=0.8 μm

λ=0.8 μmλT=2445μmKT=3056 K

15-26 Radiation emitted by a light source is maximum in the blue range. The temperature of this light source and the fraction of radiation it emits in the visible range are to be determined.

Assumptions The light source behaves as a black body.

Analysis The temperature of this light source is

T = ?

(λT)max power=2897.8μm⋅KT=2897.8μm⋅K0.47μm=6166 K

The visible range of the electromagnetic spectrum extends from λ1=0.40μm to λ2=0.76μm .  Noting that T = 6166 K, the blackbody radiation functions corresponding to λ1T and λ2T are determined from Table 15-2 to be

λ1T=(0.40μm)(6166 K)=2466 μmKfλ1=0.15440λ2T=(0.76μm)(6166 K)=4686 μmKfλ2=0.59144

Then the fraction of radiation emitted between these two wavelengths becomes

fλ2−fλ1=0.59144−0.15440≃0.437     (or 43.7%)

15-27 A glass window transmits 90% of the radiation in a specified wavelength range and is opaque for radiation at other wavelengths. The rate of radiation transmitted through this window is to be determined for two cases.

Glass

 = 0.9

L = 2 m

SUN

Assumptions The sources behave as a black body.

Analysis The surface area of the glass window is

As=4 m2

(a) For a blackbody source at 5800 K, the total blackbody radiation emission is

Eb(T)=σT4As=(5.67×10−8 kW/m2.K)4(5800 K)4(4 m2)=2.567×105 kW

The fraction of radiation in the range of 0.3 to 3.0 m is

λ1T=(0.30μm)(5800 K)=1740 μmKfλ1=0.03345λ2T=(3.0μm)(5800 K)=17,400 μmKfλ2=0.97875

Δf=fλ2−fλ1=0.97875−0.03345=0.9453

Noting that 90% of the total radiation is transmitted through the window,

Etransmit=0.90ΔfEb(T)(0.90)(0.9453)(2.567×105 kW)=2.184×105 kW

(b) For a blackbody source at 1000 K, the total blackbody emissive power is

Eb(T)=σT4As=(5.67×10−8 W/m2.K4)(1000 K)4(4 m2)=226.8 kW

The fraction of radiation in the visible range of 0.3 to 3.0 m is

λ1T=(0.30μm)(1000 K)=300 μmKfλ1=0.0000λ2T=(3.0μm)(1000 K)=3000 μmKfλ2=0.273232

Δf=fλ2−fλ1=0.273232−0

and

Etransmit=0.90ΔfEb(T)=(0.90)(0.273232)(226.8 kW)=55.8 kW

# Radiation Properties

15-28C The emissivity  is the ratio of the radiation emitted by the surface to the radiation emitted by a blackbody at the same temperature. The fraction of radiation absorbed by the surface is called the absorptivity ,

ε(T)=E(T)Eb(T)   and   α=absorbed radiationincident radiation=GabsG

When the surface temperature is equal to the temperature of the source of radiation, the total hemispherical emissivity of a surface at temperature T is equal to its total hemispherical absorptivity for radiation coming from a blackbody at the same temperature ελ(T)=αλ(T).

15-29C The fraction of irradiation reflected by the surface is called reflectivity ρ and the fraction transmitted is called the transmissivity τ

ρ=GrefG   and   τ=GtrG

Surfaces are assumed to reflect in a perfectly spectral or diffuse manner for simplicity. In spectral (or mirror like) reflection, the angle of reflection equals the angle of incidence of the radiation beam. In diffuse reflection, radiation is reflected equally in all directions.

15-30C A body whose surface properties are independent of wavelength is said to be a graybody. The emissivity of a blackbody is one for all wavelengths, the emissivity of a graybody is between zero and one.

15-31C The heating effect which is due to the non-gray characteristic of glass, clear plastic, or atmospheric gases is known as the greenhouse effect since this effect is utilized primarily in greenhouses. The combustion gases such as CO2 and water vapor in the atmosphere transmit the bulk of the solar radiation but absorb the infrared radiation emitted by the surface of the earth, acting like a heat trap. There is a concern that the energy trapped on earth will eventually cause global warming and thus drastic changes in weather patterns.

15-32C Glass has a transparent window in the wavelength range 0.3 to 3 m and it is not transparent to the radiation which has wavelength range greater than 3 m. Therefore, because the microwaves are in the range of 102 to 105 m, the harmful microwave radiation cannot escape from the glass door.

15-33 The variation of emissivity of a surface at a specified temperature with wavelength is given. The average emissivity of the surface and its emissive power are to be determined.



, m

2

6

0.7

0.4

0.3

Analysis The average emissivity of the surface can be determined from

ε(T)=ε10λ1Ebλ(T)dλσT4+ε2λ1λ2Ebλ(T)dλσT4+ε3λ2∞Ebλ(T)dλσT4       =ε1f0-λ1+ε2fλ1−λ2+ε3fλ2−∞       =ε1fλ1+ε2(fλ2−fλ1)+ε3(1−fλ2)

where  are blackbody radiation functions corresponding to λ1T and λ2T, determined from

λ1T=(2μm)(1000 K)=2000 μmKfλ1=0.066728λ2T=(6μm)(1000 K)=6000 μmKfλ2=0.737818

f0−λ1=fλ1−f0=fλ1 since f0=0 and fλ2−∞=f∞−fλ2 since f∞=1.

and,

ε=(0.4)0.066728+(0.7)(0.737818−0.066728)+(0.3)(1−0.737818)=0.575

Then the emissive power of the surface becomes

E=εσT4=0.575(5.67×10−8 W/m2.K4)(1000 K)4=32.6 kW/m2

15-34 The variation of reflectivity of a surface with wavelength is given. The average reflectivity, emissivity, and absorptivity of the surface are to be determined for two source temperatures.



, m

3

0.95

0.35

Analysis The average reflectivity of this surface for solar radiation (T = 5800 K) is determined to be

λT=(3μm)(5800 K)=17400 μmK→fλ=0.978746

ρ(T)=ρ1f0−λ1(T)+ρ2fλ1−∞(T)        =ρ1fλ1+ρ2(1−fλ1)        =(0.35)(0.978746)+(0.95)(1−0.978746)=0.362

Noting that this is an opaque surface, τ=0

At T = 5800 K:	α+ρ=1α=1−ρ=1−0.362=0.638

Repeating calculations for radiation coming from surfaces at T = 300 K,

λT=(3μm)(300 K)=900 μmKfλ1=0.0001685

ρ(T)=(0.35)(0.0001685)+(0.95)(1−0.0001685)=0.95

At T = 300 K:	α+ρ=1α=1−ρ=1−0.95=0.05

and	ε=α=0.05

The temperature of the aluminum plate is close to room temperature, and thus emissivity of the plate will be equal to its absorptivity at room temperature. That is,

ε=εroom=0.05α=αs=0.638

which makes it suitable as a solar collector. (αs=1 and εroom=0 for an ideal solar collector)



, m

3

0.7

15-35 The variation of transmissivity of the glass window of a furnace at a specified temperature with wavelength is given. The fraction and the rate of radiation coming from the furnace and transmitted through the window are to be determined.

Assumptions The window glass behaves as a black body.

Analysis The fraction of radiation at wavelengths smaller than 3 m is

λT=(3μm)(1200 K)=3600 μmKfλ=0.403607

The fraction of radiation coming from the furnace and transmitted through the window is

τ(T)=τ1fλ+τ2(1−fλ)        =(0.7)(0.403607)+(0)(1−0.403607)0.2825

Then the rate of radiation coming from the furnace and transmitted through the window becomes

Gtr=τAσT4=0.2825(0.40×0.40 m2)(5.67×10−8 W/m2.K4)(1200 K)4=5315 W

15-36 The variation of emissivity of a tungsten filament with wavelength is given. The average emissivity, absorptivity, and reflectivity of the filament are to be determined for two temperatures.

Analysis (a) T = 2000 K

λ1T=(1μm)(2000 K)=2000 μmKfλ1=0.066728



, m

1

0.5

0.15

The average emissivity of this surface is

ε(T)=ε1fλ1+ε2(1−fλ1)        =(0.5)(0.066728)+(0.15)(1−0.066728)=0.173

From Kirchhoff’s law,

ε=α=0.173   (at 2000 K)

and	α+ρ=1ρ=1−α=1−0.173=0.827

(b) T = 3000 K

λ1T=(1μm)(3000 K)=3000 μmKfλ1=0.273232

Then

ε(T)=ε1fλ1+ε2(1−fλ1)=(0.5)(0.273232)+(0.15)(1−0.273232)=0.246

From Kirchhoff’s law,

ε=α=0.246   (at 3000 K)

and	α+ρ=1ρ=1−α=1−0.246=0.754

15-37 The variations of emissivity of two surfaces are given. The average emissivity, absorptivity, and reflectivity of each surface are to be determined at the given temperature.

Analysis For the first surface:



, m

3

0.8

0.2

0.1

0.9

λ1T=(3μm)(3000 K)=9000 μmKfλ1=0.890029

The average emissivity of this surface is

ε(T)=ε1fλ1+ε2(1−fλ1)        =(0.2)(0.890029)+(0.9)(1−0.890029)=0.28

The absorptivity and reflectivity are determined from Kirchhoff’s law

ε=α=0.28     (at 3000 K)α+ρ=1ρ=1−α=1−0.28=0.72

For the second surface:

λ1T=(3μm)(3000 K)=9000 μmKfλ1=0.890029

The average emissivity of this surface is

ε(T)=ε1fλ1+ε2(1−fλ1)        =(0.8)(0.890029)+(0.1)(1−0.890029)=0.72

Then,

ε=α=0.72     (at 3000 K)α+ρ=1→ρ=1−α=1−0.72=0.28

Discussion The second surface is more suitable to serve as a solar absorber since its absorptivity for short wavelength radiation (typical of radiation emitted by a high-temperature source such as the sun) is high, and its emissivity for long wavelength radiation (typical of emitted radiation from the absorber plate) is low.

15-38 The variation of emissivity of a surface with wavelength is given. The average emissivity and  absorptivity of the surface are to be determined for two temperatures.

Analysis (a) For T = 5800 K:

λ1T=(5μm)(5800 K)=29,000 μmKfλ1=0.994715

0.9

0.15



, m

5

The average emissivity of this surface is

ε(T)=ε1fλ1+ε2(1−fλ1)        =(0.15)(0.994715)+(0.9)(1−0.994715)=0.154

(b) For T = 300 K:

λ1T=(5μm)(300 K)=1500 μmKfλ1=0.013754

and

ε(T)=ε1fλ1+ε2(1−fλ1)        =(0.15)(0.013754)+(0.9)(1−0.013754)=0.89

The absorptivities of this surface for radiation coming from sources at 5800 K and 300 K are, from Kirchhoff’s law,

α=ε=0.154	(at 5800 K)

α=ε=0.89	(at 300 K)

15-39 The variation of absorptivity of a surface with wavelength is given. The average absorptivity, reflectivity, and emissivity of the surface are to be determined at given temperatures.

Analysis For T = 2500 K:

0.7

0.2



, m

2

λ1T=(2μm)(2500 K)=5000 μmKfλ1=0.633747

The average absorptivity of this surface is

α(T)=α1fλ1+α2(1−fλ1)        =(0.2)(0.633747)+(0.7)(1−0.633747)=0.38

Then the reflectivity of this surface becomes

α+ρ=1ρ=1−α=1−0.38=0.62

Using Kirchhoff’s law, α=ε, the average emissivity of this surface at T  = 3000 K is determined to be

λT=(2μm)(3000 K)=6000 μmKfλ=0.737818

ε(T)=ε1fλ1+ε2(1−fλ1)        =(0.2)(0.737818)+(0.7)(1−0.737818)=0.33

15-40E A spherical ball emits radiation at a certain rate. The average emissivity of the ball is to be determined at the given temperature.

Ball

T=950 R

D = 5 in

Analysis The surface area of the ball is

τ(T)=τ1(fλ2−fλ1)=(0.9)(0.0001685−0.0)=0.00015≈0

Then the average emissivity of the ball at this temperature is determined to be

E=εAσT4ε=EAσT4=550 Btu/h(0.5454 ft2)(0.1714×10-8 Btu/h.ft2⋅R4)(950 R)4=0.722

15-41 The variation of transmissivity of a glass is given. The average transmissivity of the pane at two temperatures and the amount of solar radiation transmitted through the pane are to be determined.

Analysis For T=5800 K:

0.92



, m

0.3

3

λ1T1=(0.3μm)(5800 K)=1740 μmK                                        fλ1=0.033454

λ2T1=(3μm)(5800 K)=17,400 μmK                                         fλ2=0.978746

The average transmissivity of this surface is

τ(T)=τ1(fλ2−fλ1)=(0.92)(0.978746−0.033454)=0.870

For T=300 K:

λ1T2=(0.3μm)(300 K)=90 μmKfλ1=0.0

λ2T2=(3μm)(300 K)=900 μmKfλ2=0.0001685

Then,

τ(T)=τ1(fλ2−fλ1)=(0.92)(0.0001685−0.0)=0.00016≈0

The amount of solar radiation transmitted through this glass is

Gtr=τGincident=0.870(650 W/m2)=566  W/m2

# View Factors

15-42C The view factor Fi→j represents the fraction of the radiation leaving surface i that strikes surface j directly. The view factor from a surface to itself is non-zero for concave surfaces.

15-43C The pair of view factors Fi→j and Fj→i are related to each other by the reciprocity rule AiFij=AjFji where Ai is the area of the surface i and Aj is the area of the surface j. Therefore,

A1F12=A2F21F12=A2A1F21

15-44C The summation rule for an enclosure and is expressed as  j=1NFi→j=1 where N is the number of surfaces of the enclosure. It states that the sum of the view factors from surface i of an enclosure to all surfaces of the enclosure, including to itself must be equal to unity.

The superposition rule is stated as the view factor from a surface i to a surface j is equal to the sum of the view factors from surface i to the parts of surface j, F1→(2,3)=F1→2+F1→3.

15-45C The cross-string method is applicable to geometries which are very long in one direction relative to the other directions. By attaching strings between corners the Crossed-Strings Method is expressed as

Fi→j=Crossed strings−Uncrossed strings2×string on surface i

15-46 An enclosure consisting of eight surfaces is considered. The number of view factors this geometry involves and the number of these view factors that can be determined by the application of the reciprocity and summation rules are to be determined.

6

5

4

3

2

1

7

8

Analysis An eight surface enclosure (N = 8) involves N2=82=64 view factors and we need to determine N(N−1)2=8(8−1)2=28 view factors directly. The remaining 64 - 28 = 36 of the view factors can be determined by the application of the reciprocity and summation rules.

2

1

4

5

3

15-47 An enclosure consisting of five surfaces is considered. The number of view factors this geometry involves and the number of these view factors that can be determined by the application of the reciprocity and summation rules are to be determined.

Analysis A five surface enclosure (N=5) involves N2=52=25 view factors and we need to determine N(N−1)2=5(5−1)2=10 view factors directly. The remaining 25-10 = 15 of the view factors can be determined by the application of the reciprocity and summation rules.

2

1

3

9

11

12

10

4

5

8

6

7

15-48 An enclosure consisting of twelve surfaces is considered. The number of view factors this geometry involves and the number of these view factors that can be determined by the application of the reciprocity and summation rules are to be determined.

Analysis A twelve surface enclosure (N=12) involves N2=122=144 view factors  and we need to determine  N(N−1)2=12(12−1)2=66 view factors directly. The remaining 144-66 = 78 of the view factors can be determined by the application of the reciprocity and summation rules.

15-49 The view factors between the rectangular surfaces shown in the figure are to be determined.

Assumptions The surfaces are diffuse emitters and reflectors.

Analysis From Fig. 15-6,

W = 3 m

(2)

L2 = 1 m

L1 = 1 m

L3 = 1 m

A3     (3)

A2

A1

(1)

L3W=13=0.33F31=0.27

and

L3W=13=0.33F3→(1+2)=0.32

We note that A1 = A3. Then the reciprocity and superposition rules gives

A1F13=A3F31F13=F31=0.27

F3→(1+2)=F31+F32     0.32=0.27+F32F32=0.05

Finally,A2=A3F23=F32=0.05

15-50 A cylindrical enclosure is considered. The view factor from the side surface of this cylindrical enclosure to its base surface is to be determined.

Assumptions The surfaces are diffuse emitters and reflectors.

Analysis We designate the surfaces as follows:

(2)

(3)

(1)

L=2D=4r

D=2r

Base surface by (1),

top surface by (2), and

side surface by (3).

Then from Fig. 15-7

Lr1=4r1r1=4F12=F21=0.05

summation rule: F11+F12+F13=1

0+0.05+F13=1F13=0.95

reciprocity rule: A1F13=A3F31F31=A1A3F13=πr122πr1LF13=πr128πr12F13=18(0.95)=0.119

Discussion This problem can be solved more accurately by using the view factor relation from Table 15-3 to be

R1=r1L=r14r1=0.25R2=r2L=r24r2=0.25

S=1+1+R22R12=1+1+0.2520.252=18F12=12S−S2−4R2R120.5=1218−182−41120.5=0.056

F13=1−F12=1−0.056=0.944

reciprocity rule: A1F13=A3F31F31=A1A3F13=πr122πr1LF13=πr128πr12F13=18(0.944)=0.118

15-51 A semispherical furnace is considered. The view factor from the dome of this furnace to its flat base is to be determined.

(1)

(2)

D

Assumptions The surfaces are diffuse emitters and reflectors.

Analysis We number the surfaces as follows:

(1):  circular base surface

(2):   dome surface

Surface (1) is flat, and thus  .

Summation rule:   F11+F12=1→F12=1

reciprocity rule:  A1F12=A2F21F21=A1A2F12=A1A2(1)=πD24πD22=12=0.5

(1)

(2)

D

15-52 Two view factors associated with three very long ducts with different geometries are to be determined.

Assumptions 1 The surfaces are diffuse emitters and reflectors. 2 End effects are neglected.

Analysis (a) Surface (1) is flat, and thus F11=0.

(1)

(3)         (2)

a

b

summation rule:   F11+F12=1→F12=1 reciprocity rule:   A1F12=A2F21F21=A1A2F12=DsπD2s(1)=2π=0.64

(b) Noting that surfaces 2 and 3 are symmetrical and thus F12=F13, the summation rule gives

F11+F12+F13=10+F12+F13=1F12=0.5

Also by using the equation obtained in Example 15-4,

F12=L1+L2−L32L1=a+b−b2a=a2a=12=0.5

L3 = b                                     L4 = b

L6

L5

L2 = a

L1 = a

reciprocity rule:  A1F12=A2F21F21=A1A2F12=ab12=a2b

(c) Applying the crossed-string method gives

F12=F21=(L5+L6)−(L3+L4)2L12a2+b2−2b2a=a2+b2−ba

15-53 View factors from the very long grooves shown in the figure to the surroundings are to be determined.

(1)

(2)

D

Assumptions 1 The surfaces are diffuse emitters and reflectors. 2 End effects are neglected.

Analysis (a) We designate the circular dome surface by (1) and the imaginary flat top surface by (2). Noting that (2) is flat,

F22=0

summation rule:   F21+F22=1F21=1 reciprocity rule: A1F12=A2F21F12=A2A1F21=DπD2(1)=2π=0.64

(b) We designate the two identical surfaces of length b by (1) and (3), and the imaginary flat top surface by (2). Noting that (2) is flat,

(3)          (1)

a

b                             b

(2)

F22=0

summation rule:  F21+F22+F23=1F21=F23=0.5   (symmetry)

summation  rule: F22+F2→(1+3)=1F2→(1+3)=1

reciprocity rule: A2F2→(1+3)=A(1+3)F(1+3)→2F(1+3)→2=F(1+3)→surr=A2A(1+3)(1)=a2b

b                                            b

(2)                       (3)

(1)

a

(4)

(c) We designate the bottom surface by (1), the side surfaces by (2) and (3), and the imaginary top surface by (4). Surface 4 is flat and is completely surrounded by other surfaces. Therefore, F44=0 and F4→(1+2+3)=1.

reciprocity rule: A4F4→(1+2+3)=A(1+2+3)F(1+2+3)→4F(1+2+3)→4=F(1+2+3)→surr=A4A(1+2+3)(1)=aa+2b

(1)

(3), (4), (5), (6)

side surfaces

(2)

15-54 The view factors from the base of a cube to each of the other five surfaces are to be determined.

Assumptions The surfaces are diffuse emitters and reflectors.

Analysis Noting that L1D=L2D=1, from Fig. 15-6 we read

F12=0.2

Because of symmetry, we have

F12=F13=F14=F15=F16=0.2

15-55 The view factor from the conical side surface to a hole located at the center of the base of a conical enclosure is to be determined.

d

D

h

(2)     (1)

(3)

Assumptions The conical side surface is diffuse emitter and reflector.

Analysis We number different surfaces as

the hole located at the center of the base (1)

the base of conical enclosure                   (2)

conical side surface                                 (3)

Surfaces 1 and 2 are flat, and they have no direct view of each other. Therefore,

F11=F22=F12=F21=0

summation rule:   F11+F12+F13=1F13=1

reciprocity rule:  A1F13=A3F31πd24(1)=πDh2F31F31=d22Dh

15-56 The four view factors associated with an enclosure formed by two very long concentric cylinders are to be determined.

Assumptions 1 The surfaces are diffuse emitters and reflectors. 2 End effects are neglected.

(2)

D2       D1

(1)

Analysis We number different surfaces as

the outer surface of the inner cylinder (1)

the inner surface of the outer cylinder (2)

No radiation leaving surface 1 strikes itself and thusF11=0

All radiation leaving surface 1 strikes surface 2 and thus F12=1

reciprocity rule:   A1F12=A2F21F21=A1A2F12=πD1hπD2h(1)=D1D2

summation rule:  F21+F22=1F22=1−F21=1−D1D2

15-57 The view factors between the rectangular surfaces shown in the figure are to be determined.

Assumptions The surfaces are diffuse emitters and reflectors.

3 m

(1)

(3)

1 m

1 m

1 m

(2)

1 m

(4)

Analysis We designate the different surfaces as follows:

shaded part of perpendicular surface by (1),

bottom part of perpendicular surface by (3),

shaded part of horizontal surface by  (2), and

front part of horizontal surface by (4).

(a) From Fig.15-6

L2W=13F23=0.25    and       L2W=23F2→(1+3)=0.32

superposition rule:  F2→(1+3)=F21+F23F21=F2→(1+3)−F23=0.32−0.25=0.07

reciprocity rule:  A1=A2A1F12=A2F21F12=F21=0.07

(b) From Fig.15-6,

L2W=13    and    L1W=23F(4+2)→3=0.15    and     L2W=23     and    L1W=23F(4+2)→(1+3)=0.22

superposition rule:  F(4+2)→(1+3)=F(4+2)→1+F(4+2)→3F(4+2)→1=0.22−0.15=0.07

3 m

(1)

(3)

1 m

1 m

1 m

1 m

(4)

(2)

reciprocity rule:  A(4+2)F(4+2)→1=A1F1→(4+2)F1→(4+2)=A(4+2)A1F(4+2)→1=63(0.07)=0.14

superposition rule:  F1→(4+2)=F14+F12F14=0.14−0.07=0.07

since  = 0.07 (from part a). Note that F14 in part (b) is equivalent to F12 in part (a).

(c) We designate

shaded part of top surface by (1),

remaining part of top surface by (3),

2 m

2 m

1 m

(1)

(4)

(2)

1 m

1 m

1 m

(3)

remaining part of bottom surface by (4), and

shaded part of bottom surface by (2).

From Fig.15-5,

L2D=22F(2+4)→(1+3)=0.20   and   L2D=22F14=0.12

superposition rule:  F(2+4)→(1+3)=F(2+4)→1+F(2+4)→3

symmetry rule:  F(2+4)→1=F(2+4)→3

Substituting symmetry rule gives

F(2+4)→1=F(2+4)→3=F(2+4)→(1+3)2=0.202=0.10

reciprocity rule:  A1F1→(2+4)=A(2+4)F(2+4)→1(2)F1→(2+4)=(4)(0.10)F1→(2+4)=0.20

superposition rule :    F1→(2+4)=F12+F140.20=F12+0.12F12=0.20−0.12=0.08

15-58 The view factor between the two infinitely long parallel cylinders located a distance s apart from each other is to be determined.

Assumptions The surfaces are diffuse emitters and reflectors.

D

D

(2)

(1)

s

Analysis Using the crossed-strings method, the view factor between two cylinders facing each other for s/D > 3 is determined to be

F1−2=Crossed strings−Uncrossed strings2×String on surface 12s2+D2−2s2(πD2)

or	F1−2=2s2+D2−sπD

D

D

(1)

s

D

(surr)

s

(2)

(2)

15-59 Three infinitely long cylinders are located parallel to each other. The view factor between the cylinder in the middle and the surroundings is to be determined.

Assumptions The cylinder surfaces are diffuse emitters and reflectors.

Analysis The view factor between two cylinder facing each other is, from Prob. 15-17,

F1−2=2s2+D2−sπD

Noting that the radiation leaving cylinder 1 that does not strike the cylinder will strike the surroundings, and this is also the case for the other half of the cylinder, the view factor between the cylinder in the middle and the surroundings becomes

F1−surr=1−2F1−2=1−4s2+D2−sπD

# Radiation Heat Transfer between Surfaces

15-60C The analysis of radiation exchange between black surfaces is relatively easy because of the absence of reflection. The rate of radiation heat transfer between two surfaces in this case is expressed as Q=A1F12σ(T14−T24) where A1 is the surface area, F12 is the view factor, and T1 and T2 are the temperatures of two surfaces.

15-61C Radiosity is the total radiation energy leaving a surface per unit time and per unit area. Radiosity includes the emitted radiation energy as well as reflected energy. Radiosity and emitted energy are equal for blackbodies since a blackbody does not reflect any radiation.

15-62C Radiation surface resistance is given as Ri=1−εiAiεi and it represents the resistance of a surface to the emission of radiation. It is zero for black surfaces. The space resistance is the radiation resistance between two surfaces and is expressed as Rij=1AiFij

15-63C The two methods used in radiation analysis are the matrix and network methods. In matrix method, equations 15-34 and 15-35 give N linear algebraic equations for the determination of the N unknown radiosities for an N -surface enclosure. Once the radiosities are available, the unknown surface temperatures and heat transfer rates can be determined from these equations respectively. This method involves the use of matrices especially when there are a large number of surfaces. Therefore this method requires some knowledge of linear algebra.

The network method involves drawing a surface resistance associated with each surface of an enclosure and connecting them with space resistances. Then the radiation problem is solved by treating it as an electrical network problem where the radiation heat transfer replaces the current and the radiosity replaces the potential. The network method is not practical for enclosures with more than three or four surfaces due to the increased complexity of the network.

15-64C Some surfaces encountered in numerous practical heat transfer applications are modeled as being adiabatic as the back sides of these surfaces are well insulated and net heat transfer through these surfaces is zero. When the convection effects on the front (heat transfer) side of such a surface is negligible and steady-state conditions are reached, the surface must lose as much radiation energy as it receives. Such a surface is called reradiating surface. In radiation analysis, the surface resistance of a reradiating surface is taken to be zero since there is no heat transfer through it.

- 15-65 A solid sphere is placed in an evacuated equilateral triangular enclosure. The view factor from the enclosure to the sphere and the emissivity of the enclosure are to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered.

T2 = 380 K

2 = ?

T1 = 500 K

1 = 0.45

1 m

2 m

2 m

2 m

Properties The emissivity of sphere is given to be  1 = 0.45.

Analysis (a) We take the sphere to be surface 1 and the surrounding enclosure to be surface 2. The view factor from surface 2 to surface 1 is determined from reciprocity relation:

A1=πD2=π(1 m)2=3.142 m2A2=3L2−D2L2=3(2 m)2−(1 m)22 m2=5.196 m2A1F12=A2F21(3.142)(1)=(5.196)F21F21=0.605

(b) The net rate of radiation heat transfer can be expressed for this two-surface enclosure to yield the emissivity of the enclosure:

Q=σT14−T241−ε1A1ε1+1A1F12+1−ε2A2ε23100 W=(5.67×10−8 W/m2⋅K4)500 K4−380 K41−0.45(3.142 m2)(0.45)+1(3.142 m2)(1)+1−ε2(5.196 m2)ε2ε2=0.78

15-66 Radiation heat transfer occurs between a sphere and a circular disk. The view factors and the net rate of radiation heat transfer for the existing and modified cases are to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered.

Properties The emissivities of sphere and disk are given to be 1 = 0.9 and 2 = 0.5, respectively.

Analysis (a) We take the sphere to be surface 1 and the disk to be surface 2. The view factor from surface 1 to surface 2 is determined from

F12=0.51−1+r2h2−0.5=0.51−1+1.2 m0.60 m2−0.5=0.2764

r1

r2

h

The view factor from surface 2 to surface 1 is determined from reciprocity relation:

A1=4πr12=4π(0.3 m)2=1.131 m2A2=πr22=π(1.2 m)2=4.524 m2A1F12=A2F21(1.131)(0.2764)=(4.524)F21F21=0.0691

(b) The net rate of radiation heat transfer between the surfaces can be determined from

Q=σT14−T241−ε1A1ε1+1A1F12+1−ε2A2ε2=(5.67×10−8 W/m2⋅K4)873 K4−473 K41−0.9(1.131 m2)(0.9)+1(1.131 m2)(0.2764)+1−0.5(4.524 m2)(0.5)=8550 W

(c) The best values are ε1=ε2=1  and  h=r1=0.3 m. Then the view factor becomes

F12=0.51−1+r2h2−0.5=0.51−1+1.2 m0.30 m2−0.5=0.3787

The net rate of radiation heat transfer in this case is

Q=A1F12σT14−T24=(1.131 m2)(0.3787)(5.67×10−8 W/m2⋅K4)873 K4−473 K4=12,890 W

15-67E Top and side surfaces of a cubical furnace are black, and are maintained at uniform temperatures. Net radiation heat transfer rate to the base from the top and side surfaces are to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered.

Properties The emissivities are given to be  = 0.7 for the bottom surface and 1 for other surfaces.

Analysis We consider the base surface to be surface 1, the top surface to be surface 2 and the side surfaces to be surface 3. The cubical furnace can be considered to be three-surface enclosure. The areas and blackbody emissive powers of surfaces are

T1 = 800 R

1 = 0.7

T2 = 1600 R

2 = 1

T3 = 2400 R

3 = 1

A1=A2=(10 ft)2=100 ft2          A3=4(10 ft)2=400 ft2

Eb1=σT14=(0.1714×10−8 Btu/h.ft2.R4)(800 R)4=702 Btu/h.ft2Eb2=σT24=(0.1714×10−8 Btu/h.ft2.R4)(1600 R)4=11,233 Btu/h.ft2Eb3=σT34=(0.1714×10−8 Btu/h.ft2.R4)(2400 R)4=56,866 Btu/h.ft2

The view factor from the base to the top surface of the cube is F12=0.2. From the summation rule, the view factor from the base or  top to the side surfaces is

F11+F12+F13=1F13=1−F12=1−0.2=0.8

since the base surface is flat and thus F11=0. Then the radiation resistances become

R1=1−ε1A1ε1=1−0.7(100 ft2)(0.7)=0.0043 ft-2          R12=1A1F12=1(100 ft2)(0.2)=0.0500 ft-2R13=1A1F13=1(100 ft2)(0.8)=0.0125 ft-2

Note that the side and the top surfaces are black, and thus their radiosities are equal to their emissive powers. The radiosity of the base surface is determined

Eb1−J1R1+Eb2−J1R12+Eb3−J1R13=0

Substituting,	702−J10.0043+11,233−J10.05+56,866−J10.0125=0J1=15,054 W/m2

(a) The net rate of radiation heat transfer between the base and the side surfaces is

Q31=Eb3−J1R13=(56,866−15,054) Btu/h.ft20.0125 ft-2=3.345×106 Btu/h

(b) The net rate of radiation heat transfer between the base and the top surfaces is

Q12=J1−Eb2R12=(15,054−11,233) Btu/h.ft20.05 ft-2=7.642×104 Btu/h

The net rate of radiation heat transfer to the base surface is finally determined from

Q1=Q21+Q31=−76,420+3,344,960=3.269×106 Btu/h

Discussion The same result can be found form

Q1=J1−Eb1R1=(15,054−702) Btu/h.ft20.0043 ft-2=3.338×106 Btu/h

The small difference is due to round-off error.

15-68E EES Prob. 15-67E is reconsidered.  The effect of base surface emissivity on the net rates of radiation heat transfer between the base and the side surfaces, between the base and top surfaces, and to the base surface is to be investigated.

Analysis The problem is solved using EES, and the solution is given below.

"GIVEN"

a=10 [ft]

epsilon_1=0.7

T_1=800 [R]

T_2=1600 [R]

T_3=2400 [R]

"ANALYSIS"

sigma=0.1714E-8 [Btu/h-ft^2-R^4] “Stefan-Boltzmann constant"

"Consider the base surface 1, the top surface 2, and the side surface 3"

E_b1=sigma*T_1^4

E_b2=sigma*T_2^4

E_b3=sigma*T_3^4

A_1=a^2

A_2=A_1

A_3=4*a^2

F_12=0.2 "view factor from the base to the top of a cube"

F_11+F_12+F_13=1 "summation rule"

F_11=0 "since the base surface is flat"

R_1=(1-epsilon_1)/(A_1*epsilon_1) "surface resistance"

R_12=1/(A_1*F_12) "space resistance"

R_13=1/(A_1*F_13) "space resistance"

(E_b1-J_1)/R_1+(E_b2-J_1)/R_12+(E_b3-J_1)/R_13=0 "J_1 : radiosity of base surface"

"(a)"

Q_dot_31=(E_b3-J_1)/R_13

"(b)"

Q_dot_12=(J_1-E_b2)/R_12

Q_dot_21=-Q_dot_12

Q_dot_1=Q_dot_21+Q_dot_31

| 1 | Q31 [Btu/h] | Q12 [Btu/h] | Q1 [Btu/h] |
| --- | --- | --- | --- |
| 0.1 | 1.106E+06 | 636061 | 470376 |
| 0.15 | 1.295E+06 | 589024 | 705565 |
| 0.2 | 1.483E+06 | 541986 | 940753 |
| 0.25 | 1.671E+06 | 494948 | 1.176E+06 |
| 0.3 | 1.859E+06 | 447911 | 1.411E+06 |
| 0.35 | 2.047E+06 | 400873 | 1.646E+06 |
| 0.4 | 2.235E+06 | 353835 | 1.882E+06 |
| 0.45 | 2.423E+06 | 306798 | 2.117E+06 |
| 0.5 | 2.612E+06 | 259760 | 2.352E+06 |
| 0.55 | 2.800E+06 | 212722 | 2.587E+06 |
| 0.6 | 2.988E+06 | 165685 | 2.822E+06 |
| 0.65 | 3.176E+06 | 118647 | 3.057E+06 |
| 0.7 | 3.364E+06 | 71610 | 3.293E+06 |
| 0.75 | 3.552E+06 | 24572 | 3.528E+06 |
| 0.8 | 3.741E+06 | -22466 | 3.763E+06 |
| 0.85 | 3.929E+06 | -69503 | 3.998E+06 |
| 0.9 | 4.117E+06 | -116541 | 4.233E+06 |

15-69 Two very large parallel plates are maintained at uniform temperatures. The net rate of radiation heat transfer between the two plates is to be determined.

T2 = 400 K

2 = 0.9

T1 = 600 K

1 = 0.5

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered.

Properties The emissivities  of the plates are given to be 0.5 and 0.9.

Analysis The net rate of radiation heat transfer between the two surfaces per unit area of the plates is determined directly from

Q12As=σ(T14−T24)1ε1+1ε2−1=(5.67×10−8 W/m2⋅K4)[(600 K)4−(400 K)4]10.5+10.9−1=2793 W/m2

15-70 EES Prob. 15-69 is reconsidered.  The effects of the temperature and the emissivity of the hot plate on the net rate of radiation heat transfer between the plates are to be investigated.

Analysis The problem is solved using EES, and the solution is given below.

"GIVEN"

T_1=600 [K]

T_2=400 [K]

epsilon_1=0.5

epsilon_2=0.9

sigma=5.67E-8 [W/m^2-K^4] “Stefan-Boltzmann constant"

"ANALYSIS"

q_dot_12=(sigma*(T_1^4-T_2^4))/(1/epsilon_1+1/epsilon_2-1)

| T1 [K] | q12 [W/m2] |
| --- | --- |
| 500 | 991.1 |
| 525 | 1353 |
| 550 | 1770 |
| 575 | 2248 |
| 600 | 2793 |
| 625 | 3411 |
| 650 | 4107 |
| 675 | 4888 |
| 700 | 5761 |
| 725 | 6733 |
| 750 | 7810 |
| 775 | 9001 |
| 800 | 10313 |
| 825 | 11754 |
| 850 | 13332 |
| 875 | 15056 |
| 900 | 16934 |
| 925 | 18975 |
| 950 | 21188 |
| 975 | 23584 |
| 1000 | 26170 |

| 1 | q12 [W/m2] |
| --- | --- |
| 0.1 | 583.2 |
| 0.15 | 870 |
| 0.2 | 1154 |
| 0.25 | 1434 |
| 0.3 | 1712 |
| 0.35 | 1987 |
| 0.4 | 2258 |
| 0.45 | 2527 |
| 0.5 | 2793 |
| 0.55 | 3056 |
| 0.6 | 3317 |
| 0.65 | 3575 |
| 0.7 | 3830 |
| 0.75 | 4082 |
| 0.8 | 4332 |
| 0.85 | 4580 |
| 0.9 | 4825 |

15-71 The base, top, and side surfaces of a furnace of cylindrical shape are black, and are maintained at uniform temperatures. The net rate of radiation heat transfer to or from the top surface is to be determined.

T1 = 700 K

1 = 1

r1 = 2 m

T2 = 1400 K

2 = 1

r2 = 2 m

T3 = 500 K

3 = 1

h =2 m

Assumptions 1 Steady operating conditions exist 2 The surfaces are black. 3 Convection heat transfer is not considered.

Properties The emissivity of all surfaces are  = 1 since they are black.

Analysis We consider the top surface to be surface 1, the base surface to be surface 2 and the side surfaces to be surface 3. The cylindrical furnace can be considered to be three-surface enclosure. We assume that steady-state conditions exist. Since all surfaces are black, the radiosities are equal to the emissive power of surfaces, and the net rate of radiation heat transfer from the top surface can be determined from

Q=A1F12σ(T14−T24)+A1F13σ(T14−T34)

and	A1=πr2=π(2 m)2=12.57 m2

The view factor from the base to the top surface of the cylinder is F12=0.38 (From Figure 15-7). The view factor from the base to the side surfaces is determined by applying the summation rule to be

F11+F12+F13=1F13=1−F12=1−0.38=0.62

Substituting,

Q=A1F12σ(T14−T24)+A1F13σ(T14−T34)(12.57 m2)(0.38)(5.67×10-8 W/m2.K4)(700 K4-500 K4)+(12.57 m2)(0.62)(5.67×10-8 W/m2.K4)(700 K4-1400 K4)−1.543×106 W=-1543 kW

Discussion The negative sign indicates that net heat transfer is to the top surface.

15-72 The base and the dome of a hemispherical furnace are maintained at uniform temperatures. The net rate of radiation heat transfer from the dome to the base surface is to be determined.

T1 = 400 K

1 = 0.7

T2 = 1000 K

2 = 1

D = 5 m

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered.

Analysis The view factor is first determined from

F11=0  (flat surface)F11+F12=1→F12=1  (summation rule)

Noting that the dome is black, net rate of radiation heat transfer from dome to the base surface can be determined from

Q21=−Q12=−εA1F12σ(T14−T24)     =−(0.7)[π(5 m)2/4 ](1)(5.67×10−8 W/m2⋅K4)[(400 K)4−(1000 K)4]7.594×105 W759 kW

The positive sign indicates that the net heat transfer is from the dome to the base surface, as expected.

15-73 Two very long concentric cylinders are maintained at uniform temperatures. The net rate of radiation heat transfer between the two cylinders is to be determined.

D2 = 0.5 m

T2 = 500 K

2 = 0.55

D1 = 0.35 m

T1 = 950 K

1 = 1

Vacuum

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered.

Properties The emissivities of surfaces are given to be 1 = 1 and 2 = 0.55.

Analysis The net rate of radiation heat transfer between the two cylinders per unit length of the cylinders is determined from

Q12=A1σ(T14−T24)1ε1+1−ε2ε2r1r2[π(0.35 m)(1 m)](5.67×10−8 W/m2⋅K4)[(950 K)4−(500 K)4]11+1−0.550.553.5529,810 W=29.81 kW

15-74 A long cylindrical rod coated with a new material is placed in an evacuated long cylindrical enclosure which is maintained at a uniform temperature. The emissivity of the coating on the rod is to be determined.

D2 = 0.1 m

T2 = 200 K

2 = 0.95

D1 = 0.01 m

T1 = 500 K

1 = ?

Vacuum

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray.

Properties The emissivity of the enclosure is given to be         2 = 0.95.

Analysis The emissivity of the coating on the rod is determined from

Q12=A1σ(T14−T24)1ε1+1−ε2ε2r1r28 W=[π(0.01 m)(1 m)](5.67×10−8 W/m2⋅K4)[500 K4−200 K4]1ε1+1−0.950.95110

which gives

1 = 0.074

15-75E The base and the dome of a long semicylindrical duct are maintained at uniform temperatures. The net rate of radiation heat transfer from the dome to the base surface is to be determined.

T1 = 550 R

1 = 0.5

T2 = 1800 R

2 = 0.9

D = 15 ft

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered.

Properties The emissivities of surfaces are given to be 1 = 0.5 and 2 = 0.9.

Analysis The view factor from the base to the dome is first determined from

F11=0  (flat surface)F11+F12=1→F12=1  (summation rule)

The net rate of radiation heat transfer from dome to the base surface can be determined from

Q21=−Q12=−σ(T14−T24)1−ε1A1ε1+1A1F12+1−ε2A2ε2=−(0.1714×10−8 Btu/h.ft2⋅R4)[(550 R)4−(1800 R)4]1−0.5(15 ft2)(0.5)+1(15 ft2)(1)+1−0.9π(15 ft)(1 ft)2(0.9)129,200 Btu/h   per ft length

The positive sign indicates that the net heat transfer is from the dome to the base surface, as expected.

15-76 Two parallel disks whose back sides are insulated are black, and are maintained at a uniform temperature. The net rate of radiation heat transfer from the disks to the environment is to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered.

Properties The emissivities of all surfaces are  = 1 since they are black.

Disk 1, T1 = 450 K, 1 = 1

Disk 2, T2 = 450 K, 2 = 1

0.40 m

Environment

T3 =300 K

1 = 1

D = 0.6 m

Analysis Both disks possess same properties and they are black. Noting that environment can also be considered to be blackbody, we can treat this geometry as a three surface enclosure. We consider the two disks to be surfaces 1 and 2 and the environment to be surface 3. Then from Figure 15-7, we read

F12=F21=0.26  F13=1−0.26=0.74     (summation rule)

The net rate of radiation heat transfer from the disks into the environment then becomes

Q3=Q13+Q23=2Q13Q3=2F13A1σ(T14−T34)     =2(0.74)[π(0.3 m)2](5.67×10−8 W/m2⋅K4)[450 K4−300 K4]781 W

15-77 A furnace shaped like a long equilateral-triangular duct is considered. The temperature of the base surface is to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered. 4 End effects are neglected.

q1 = 800 W/m2

1 = 0.8

T2 = 500 K

2 = 0.5

b = 2 m

Properties The emissivities of surfaces are given to be  1 = 0.8 and 2 = 0.5.

Analysis This geometry can be treated as a two surface enclosure since two surfaces have identical properties. We consider base surface to be surface 1 and other two surface to be surface 2. Then the view factor between the two becomes F12=1. The temperature of the base surface is determined from

Q12=σ(T14−T24)1−ε1A1ε1+1A1F12+1−ε2A2ε2800 W=(5.67×10−8 W/m2⋅K4)[T14−500 K4]1−0.8(1 m2)(0.8)+1(1 m2)(1)+1−0.5(2 m2)(0.5)T1=543 K

Note that A1=1 m2  and  A2=2 m2.

15-78 EES Prob. 15-77 is reconsidered.  The effects of the rate of the heat transfer at the base surface and the temperature of the side surfaces on the temperature of the base surface are to be investigated.

Analysis The problem is solved using EES, and the solution is given below.

"GIVEN"

a=2 [m]

epsilon_1=0.8

epsilon_2=0.5

Q_dot_12=800 [W]

T_2=500 [K]

sigma=5.67E-8 [W/m^2-K^4]

"ANALYSIS"

"Consider the base surface to be surface 1, the side surfaces to be surface 2"

Q_dot_12=(sigma*(T_1^4-T_2^4))/((1-epsilon_1)/(A_1*epsilon_1)+1/(A_1*F_12)+(1-epsilon_2)/(A_2*epsilon_2))

F_12=1

A_1=1 "[m^2], since rate of heat supply is given per meter square area"

A_2=2*A_1

| Q12 [W] | T1 [K] |
| --- | --- |
| 500 | 528.4 |
| 525 | 529.7 |
| 550 | 531 |
| 575 | 532.2 |
| 600 | 533.5 |
| 625 | 534.8 |
| 650 | 536 |
| 675 | 537.3 |
| 700 | 538.5 |
| 725 | 539.8 |
| 750 | 541 |
| 775 | 542.2 |
| 800 | 543.4 |
| 825 | 544.6 |
| 850 | 545.8 |
| 875 | 547 |
| 900 | 548.1 |
| 925 | 549.3 |
| 950 | 550.5 |
| 975 | 551.6 |
| 1000 | 552.8 |

| T2 [K] | T1 [K] |
| --- | --- |
| 300 | 425.5 |
| 325 | 435.1 |
| 350 | 446.4 |
| 375 | 459.2 |
| 400 | 473.6 |
| 425 | 489.3 |
| 450 | 506.3 |
| 475 | 524.4 |
| 500 | 543.4 |
| 525 | 563.3 |
| 550 | 583.8 |
| 575 | 605 |
| 600 | 626.7 |
| 625 | 648.9 |
| 650 | 671.4 |
| 675 | 694.2 |
| 700 | 717.3 |

15-79 The floor and the ceiling of a cubical furnace are maintained at uniform temperatures. The net rate of radiation heat transfer between the floor and the ceiling is to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered.

Properties The emissivities of all surfaces are   = 1 since they are black or reradiating.

Analysis We consider the ceiling to be surface 1, the floor to be surface 2 and the side surfaces to be surface 3. The furnace can be considered to be three-surface enclosure. We assume that steady-state conditions exist. Since the side surfaces are reradiating, there is no heat transfer through them, and the entire heat lost by the ceiling must be gained by the floor. The view factor from the ceiling to the floor of the furnace is F12=0.2. Then the rate of heat loss from the ceiling can be determined from

T2 = 550 K

2 = 1

T1 = 1100 K

1 = 1

Reradiating side surfacess

a = 4 m

Q1=Eb1−Eb21R12+1R13+R23−1

where

Eb1=σT14=(5.67×10−8 W/m2.K4)(1100 K)4=83,015 W/m2Eb2=σT24=(5.67×10−8 W/m2.K4)(550 K)4=5188 W/m2

and

A1=A2=(4 m)2=16 m2

R12=1A1F12=1(16 m2)(0.2)=0.3125 m-2R13=R23=1A1F13=1(16 m2)(0.8)=0.078125 m-2

Substituting,

Q12=(83,015−5188) W/m210.3125 m-2+12(0.078125 m-2)−1=7.47×105 W=747 kW

15-80 Two concentric spheres are maintained at uniform temperatures. The net rate of radiation heat transfer between the two spheres and the convection heat transfer coefficient at the outer surface are to be determined.

D2 = 0.4 m

T2 = 500 K

2 = 0.7

D1 = 0.3 m

T1 = 700 K

1 = 0.5

Tsurr =30C

T = 30C

 = 0.35

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray.

Properties The emissivities of surfaces are given to be 1 = 0.5 and 2 = 0.7.

Analysis The net rate of radiation heat transfer between the two spheres is

Q12=A1σT14−T241ε1+1−ε2ε2r12r22π(0.3 m)25.67×10−8 W/m2⋅K4700 K4−500 K410.5+1−0.70.70.15 m0.2 m21270 W

Radiation heat transfer rate from the outer sphere to the surrounding surfaces are

Qrad=εFA2σ(T24−Tsurr4)    =(0.35)(1)[π(0.4 m)2](5.67×10−8 W/m2⋅K4)[(500 K)4−(30+273 K)4]539W

The convection heat transfer rate at the outer surface of the cylinder is determined from requirement that heat transferred from the inner sphere to the outer sphere must be equal to the heat transfer from the outer surface of the outer sphere to the environment by convection and radiation. That is,

Qconv=Q12−Qrad=1270−539=731 W

Then the convection heat transfer coefficient becomes

Qconv.=hA2T2−T∞731 W=hπ(0.4 m)2(500 K-303 K)h=7.4 W/m2⋅°C

15-81 A spherical tank filled with liquid nitrogen is kept in an evacuated cubic enclosure. The net rate of radiation heat transfer to the liquid nitrogen is to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered. 4 The thermal resistance of the tank is negligible.

Properties The emissivities of surfaces are given to be  1 = 0.1 and 2 = 0.8.

Cube, a =3 m

T2 = 240 K

2 = 0.8

D1 = 2 m

T1 = 100 K

1 = 0.1

Vacuum

Liquid

N2

Analysis We take the sphere to be surface 1 and the surrounding cubic enclosure to be surface 2. Noting that F12=1, for this two-surface enclosure, the net rate of radiation heat transfer to liquid nitrogen can be determined from

Q21=−Q12=−A1σT14−T241ε1+1−ε2ε2A1A2−π(2 m)25.67×10−8 W/m2⋅K4100 K4−240 K410.1+1−0.80.8π(2 m)26(3 m)2228 W

15-82 A spherical tank filled with liquid nitrogen is kept in an evacuated spherical enclosure. The net rate of radiation heat transfer to the liquid nitrogen is to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered. 4 The thermal resistance of the tank is negligible.

D2 = 3 m

T2 = 240 K

2 = 0.8

D1 = 2 m

T1 = 100 K

1 = 0.1

Vacuum

Liquid

N2

Properties The emissivities of surfaces are given to be  1 = 0.1 and 2 = 0.8.

Analysis The net rate of radiation heat transfer to liquid nitrogen can be determined from

Q12=A1σT14−T241ε1+1−ε2ε2r12r22π(2 m)25.67×10−8 W/m2⋅K4240 K4−100 K410.1+1−0.80.8(1 m)2(1.5 m)2227 W

15-83 EES Prob. 15-81 is reconsidered.  The effects of the side length and the emissivity of the cubic enclosure, and the emissivity of the spherical tank on the net rate of radiation heat transfer are to be investigated.

Analysis The problem is solved using EES, and the solution is given below.

"GIVEN"

D=2 [m]

a=3 [m]

T_1=100 [K]

T_2=240 [K]

epsilon_1=0.1

epsilon_2=0.8

sigma=5.67E-8 [W/m^2-K^4] “Stefan-Boltzmann constant"

"ANALYSIS"

"Consider the sphere to be surface 1, the surrounding cubic enclosure to be surface 2"

Q_dot_12=(A_1*sigma*(T_1^4-T_2^4))/(1/epsilon_1+(1-epsilon_2)/epsilon_2*(A_1/A_2))

Q_dot_21=-Q_dot_12

A_1=pi*D^2

A_2=6*a^2

| a [m] | Q21 [W] |
| --- | --- |
| 2.5 | 227.4 |
| 2.625 | 227.5 |
| 2.75 | 227.7 |
| 2.875 | 227.8 |
| 3 | 227.9 |
| 3.125 | 228 |
| 3.25 | 228.1 |
| 3.375 | 228.2 |
| 3.5 | 228.3 |
| 3.625 | 228.4 |
| 3.75 | 228.4 |
| 3.875 | 228.5 |
| 4 | 228.5 |
| 4.125 | 228.6 |
| 4.25 | 228.6 |
| 4.375 | 228.6 |
| 4.5 | 228.7 |
| 4.625 | 228.7 |
| 4.75 | 228.7 |
| 4.875 | 228.8 |
| 5 | 228.8 |

| 1 | Q21 [W] |
| --- | --- |
| 0.1 | 227.9 |
| 0.15 | 340.9 |
| 0.2 | 453.3 |
| 0.25 | 565 |
| 0.3 | 676 |
| 0.35 | 786.4 |
| 0.4 | 896.2 |
| 0.45 | 1005 |
| 0.5 | 1114 |
| 0.55 | 1222 |
| 0.6 | 1329 |
| 0.65 | 1436 |
| 0.7 | 1542 |
| 0.75 | 1648 |
| 0.8 | 1753 |
| 0.85 | 1857 |
| 0.9 | 1961 |

| 2 | Q21 [W] |
| --- | --- |
| 0.1 | 189.6 |
| 0.15 | 202.6 |
| 0.2 | 209.7 |
| 0.25 | 214.3 |
| 0.3 | 217.5 |
| 0.35 | 219.8 |
| 0.4 | 221.5 |
| 0.45 | 222.9 |
| 0.5 | 224.1 |
| 0.55 | 225 |
| 0.6 | 225.8 |
| 0.65 | 226.4 |
| 0.7 | 227 |
| 0.75 | 227.5 |
| 0.8 | 227.9 |
| 0.85 | 228.3 |
| 0.9 | 228.7 |

15-84 A circular grill is considered. The bottom of the grill is covered with hot coal bricks, while the wire mesh on top of the grill is covered with steaks. The initial rate of radiation heat transfer from coal bricks to the steaks is to be determined for two cases.

Steaks, T2 = 278 K, 2 = 1

Coal bricks, T1 = 950 K, 1 = 1

0.20 m

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered.

Properties The emissivities are   = 1 for all surfaces since they are black or reradiating.

Analysis We consider the coal bricks to be surface 1, the steaks to be surface 2 and the side surfaces to be surface 3. First we determine the view factor between the bricks and the steaks (Table 15-3),

Ri=Rj=riL=0.15 m0.20 m=0.75

S=1+1+Rj2Ri2=1+0.7520.752=3.7778

F12=Fij=12S−S2−4RjRi212=123.7778−3.77782−40.750.75212=0.2864

(It can also be determined from Fig. 15-7).

Then the initial rate of radiation heat transfer from the coal bricks to the stakes becomes

Q12=F12A1σ(T14−T24)(0.2864)[π(0.3 m)24](5.67×10−8 W/m2⋅K4)[(950 K)4−(278 K)4]928 W

When the side opening is closed with aluminum foil, the entire heat lost by the coal bricks must be gained by the stakes since there will be no heat transfer through a reradiating surface. The grill can be considered to be three-surface enclosure. Then the rate of heat loss from the coal bricks can be determined from

Q1=Eb1−Eb21R12+1R13+R23−1

where	        Eb1=σT14=(5.67×10−8 W/m2.K4)(950 K)4=46,183 W/m2Eb2=σT24=(5.67×10−8 W/m2.K4)(5+273 K)4=339 W/m2

and	A1=A2=π(0.3 m)24=0.07069 m2

R12=1A1F12=1(0.07069 m2)(0.2864)=49.39 m-2R13=R23=1A1F13=1(0.07069 m2)(1−0.2864)=19.82 m-2

Substituting,	Q12=(46,183−339) W/m2149.39 m-2+12(19.82 m-2)−1=2085 W

15-85E A room is heated by electric resistance heaters placed on the ceiling which is maintained at a uniform temperature. The rate of heat loss from the room through the floor is to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered. 4 There is no heat loss through the side surfaces.

Properties The emissivities are  = 1 for the ceiling and  = 0.8 for the floor. The emissivity of insulated (or reradiating) surfaces is also 1.

T2 = 65F

2 = 0.8

Ceiling: 12 ft  12 ft

T1 = 90F

1 = 1

Insulated side surfacess

9 ft

Analysis The room can be considered to be three-surface enclosure with the ceiling surface 1, the floor surface 2 and the side surfaces surface 3. We assume steady-state conditions exist. Since the side surfaces are reradiating, there is no heat transfer through them, and the entire heat lost by the ceiling must be gained by the floor. Then the rate of heat loss from the room through its floor can be determined from

Q1=Eb1−Eb21R12+1R13+R23−1+R2

where

Eb1=σT14=(0.1714×10−8 Btu/h.ft2.R4)(90+460 R)4=157 Btu/h.ft2Eb2=σT24=(0.1714×10−8 Btu/h.ft2.R4)(65+460 R)4=130 Btu/h.ft2

and

A1=A2=(12 ft)2=144 ft2

The view factor from the floor to the ceiling of the room is F12=0.27 (From Figure 15-5). The view factor from the ceiling or the floor to the side surfaces is determined by applying the summation rule to be

F11+F12+F13=1F13=1−F12=1−0.27=0.73

since the ceiling is flat and thus F11=0. Then the radiation resistances which appear in the equation above become

R2=1−ε2A2ε2=1−0.8(144 ft2)(0.8)=0.00174 ft-2R12=1A1F12=1(144 ft2)(0.27)=0.02572 ft-2R13=R23=1A1F13=1(144 ft2)(0.73)=0.009513 ft-2

Substituting,

Q12=(157−130) Btu/h.ft210.02572 ft-2+12(0.009513 ft-2)−1+0.00174 ft-2=2130 Btu/h

15-86 Two perpendicular rectangular surfaces with a common edge are maintained at specified temperatures. The net rate of radiation heat transfers between the two surfaces and between the horizontal surface and the surroundings are to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered.

Properties The emissivities of the horizontal rectangle and the surroundings are  = 0.75 and  = 0.85, respectively.

Analysis We consider the horizontal rectangle to be surface 1, the vertical rectangle to be surface 2 and the surroundings to be surface 3. This system can be considered to be a three-surface enclosure. The view factor from surface 1 to surface 2 is determined from

W = 1.6 m

(2)

L2 = 1.2 m

L1 = 0.8 m

A1     (1)

A2

T3 = 290 K

3 = 0.85

T2 = 550 K

2 = 1

T1 =400 K

1 =0.75

(3)

L1W=0.81.6=0.5F12=0.27 (Fig. 15-6)

The surface areas are

A1=(0.8 m)(1.6 m)=1.28 m2A2=(1.2 m)(1.6 m)=1.92 m2

A3=2×1.2×0.82+0.82+1.22×1.6=3.268 m2

Note that the surface area of the surroundings is determined assuming that surroundings forms flat surfaces at all openings to form an enclosure. Then other view factors are determined to be

A1F12=A2F21(1.28)(0.27)=(1.92)F21F21=0.18		(reciprocity rule)

F11+F12+F13=10+0.27+F13=1F13=0.73		(summation rule)

F21+F22+F23=10.18+0+F23=1F23=0.82		(summation rule)

A1F13=A3F31(1.28)(0.73)=(3.268)F31F31=0.29	(reciprocity rule)

A2F23=A3F32(1.92)(0.82)=(3.268)F32F32=0.48	(reciprocity rule)

We now apply Eq. 15-35b to each surface to determine the radiosities.

Surface 1:	        σT14=J1+1−ε1ε1F12(J1−J2)+F13(J1−J3)(5.67×10−8 W/m2.K4)(400 K)4=J1+1−0.750.750.27(J1−J2)+0.73(J1−J3)

Surface 2:	        σT24=J2(5.67×10−8 W/m2.K4)(550 K)4=J2

Surface 3:	        σT34=J3+1−ε3ε3F31(J3−J1)+F32(J3−J2)(5.67×10−8 W/m2.K4)(290 K)4=J3+1−0.850.850.29(J3−J1)+0.48(J3−J2)

Solving the above equations, we find

J1=1587 W/m2,   J2=5188 W/m2,    J3=811.5 W/m2

Then the net rate of radiation heat transfers between the two surfaces and between the horizontal surface and the surroundings are determined to be

Q21=−Q12=−A1F12(J1−J2)=−(1.28 m2)(0.27)(1587−5188)W/m2=1245 W

Q13=A1F13(J1−J3)=(1.28 m2)(0.73)(1587−811.5)W/m2=725 W

15-87 Two long parallel cylinders are maintained at specified temperatures. The rates of radiation heat transfer between the cylinders and between the hot cylinder and the surroundings are to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are black. 3 Convection heat transfer is not considered.

Analysis We consider the hot cylinder to be surface 1, cold cylinder to be surface 2, and the surroundings to be surface 3. Using the crossed-strings method, the view factor between two cylinders facing each other is determined to be

D

D

(2)

(1)

s

T3 = 300 K

3 = 1

T2 = 275 K

2 = 1

T1 = 425 K

1 = 1

(3)

F1−2=Crossed strings−Uncrossed strings2×String on surface 12s2+D2−2s2(πD2)

or

F1−2=2s2+D2−sπD20.32+0.202−0.5π(0.20)0.444

The view factor between the hot cylinder and the surroundings is

F13=1−F12=1−0.444=0.556(summation rule)

The rate of radiation heat transfer between the cylinders per meter length is

A=πDL2=π(0.20 m)(1 m)2=0.3142 m2

Q12=AF12σ(T14−T24)(0.3142 m2)(0.444)(5.67×10−8 W/m2.°C)(4254−2754)K4212.8 W

Note that half of the surface area of the cylinder is used, which is the only area that faces the other cylinder. The rate of radiation heat transfer between the hot cylinder and the surroundings per meter length of the cylinder is

A1=πDL=π(0.20 m)(1 m)=0.6283 m2

Q13=A1F13σ(T14−T34)(0.6283 m2)(0.556)(5.67×10−8 W/m2.°C)(4254−3004)K4485.8 W

15-88 A long semi-cylindrical duct with specified temperature on the side surface is considered. The temperature of the base surface for a specified heat transfer rate is to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered.

Properties The emissivity of the side surface is  = 0.4.

Analysis We consider the base surface to be surface 1, the side surface to be surface 2. This system is a two-surface enclosure, and we consider a unit length of the duct. The surface areas and the view factor are determined as

T1 = ?

1 = 1

T2 = 650 K

2 = 0.4

D = 1 m

A1=(1.0 m)(1.0 m)=1.0 m2A2=πDL2=π(1.0 m)(1 m)2=1.571 m2

F11+F12=10+F12=1F12=1  (summation rule)

The temperature of the base surface is determined from

Q12=σ(T14−T24)1A1F12+1−ε2A2ε21200 W=(5.67×10−8 W/m2⋅K4)[T14−(650 K)4]1(1.0 m2)(1)+1−0.4(1.571 m2)(0.4)T1=684.8 K

15-89 A hemisphere with specified base and dome temperatures and heat transfer rate is considered. The emissivity of the dome is to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered.

Properties The emissivity of the base surface is  = 0.55.

Analysis We consider the base surface to be surface 1, the dome surface to be surface 2. This system is a two-surface enclosure. The surface areas and the view factor are determined as

T1 = 400 K

1 = 0.55

T2 = 600 K

2 = ?

D = 0.2 m

A1=πD24=π(0.2 m)24=0.0314 m2A2=πD22=π(0.2 m)22=0.0628 m2

F11+F12=10+F12=1F12=1  (summation rule)

The emissivity of the dome is determined from

Q21=−Q12=−σ(T14−T24)1−ε1A1ε1+1A1F12+1−ε2A2ε250 W=−(5.67×10−8 W/m2⋅K4)[(400 K)4−(600 K)4]1−0.55(0.0314 m2)(0.55)+1(0.0314 m2)(1)+1−ε2(0.0628 m2)ε2ε2=0.21

Review Problems

15-90 The variation of emissivity of an opaque surface at a specified temperature with wavelength is given. The average emissivity of the surface and its emissive power are to be determined.



, m

2

6

0.85

0

Analysis The average emissivity of the surface can be determined from

ε(T)=ε1fλ1+ε2(fλ2−fλ1)+ε3(1−fλ2)

where fλ1 and fλ2 are blackbody radiation functions corresponding to λ1T and λ2T. These functions are determined from Table 15-2 to be

λ1T=(2μm)(1500 K)=3000 μmKfλ1=0.273232λ2T=(6μm)(1500 K)=9000 μmKfλ2=0.890029

and

ε=(0.0)(0.273232)+(0.85)(0.890029−0.273232)+(0.0)(1−0.890029)=0.5243

Then the emissive flux of the surface becomes

E=εσT4=(0.5243)(5.67×10−8 W/m2.K4)(1500 K)4=150,500 W/m2

15-91 The variation of transmissivity of glass with wavelength is given. The transmissivity of the glass for solar radiation and for light are to be determined.



, m

0.35

2.5

0.85

0

Analysis For solar radiation, T = 5800 K. The average transmissivity of the surface can be determined from

τ(T)=τ1fλ1+τ2(fλ2−fλ1)+τ3(1−fλ2)

where fλ1 and fλ2 are blackbody radiation functions corresponding to λ1T and λ2T. These functions are determined from Table 15-2 to be

λ1T=(0.35μm)(5800 K)=2030 μmKfλ1=0.071852λ2T=(2.5μm)(5800 K)=14,500 μmKfλ2=0.966440

and

τ=(0.0)(0.071852)+(0.85)(0.966440−0.071852)+(0.0)(1−0.966440)=0.760

For light, we take T = 300 K. Repeating the calculations at this temperature we obtain

λ1T=(0.35μm)(300 K)=105 μmKfλ1=0.00λ2T=(2.5μm)(300 K)=750 μmKfλ2=0.000012

τ=(0.0)(0.00)+(0.85)(0.000012−0.00)+(0.0)(1−0.000012)=0.00001

15-92 A hole is drilled in a spherical cavity. The maximum rate of radiation energy streaming through the hole is to be determined.

Analysis The maximum rate of radiation energy streaming through the hole is the blackbody radiation, and it can be determined from

E=AσT4=π(0.0025 m)2(5.67×10−8 W/m2.K4)(600 K)4=0.144 W

The result would not change for a different diameter of the cavity.



, m

0.3

1.2

0.8

0.1

0

15-93 The variation of absorptivity of a surface with wavelength is given. The average absorptivity of the surface is to be determined for two source temperatures.

Analysis (a) T = 1000 K. The average absorptivity of the surface can be determined from

α(T)=α1f0-λ1+α2fλ1−λ2+α3fλ2−∞=α1fλ1+α2(fλ2−fλ1)+α3(1−fλ2)

where fλ1 and fλ2 are blackbody radiation functions corresponding to λ1T and λ2T, determined from

λ1T=(0.3μm)(1000 K)=300 μmKfλ1=0.0λ2T=(1.2μm)(1000 K)=1200 μmKfλ2=0.002134

f0−λ1=fλ1−f0=fλ1 since f0=0 and fλ2−∞=f∞−fλ2 since f∞=1.

and,

α=(0.1)0.0+(0.8)(0.002134−0.0)+(0.0)(1−0.002134)=0.0017

(a) T = 3000 K.

λ1T=(0.3μm)(3000 K)=900 μmKfλ1=0.000169λ2T=(1.2μm)(3000 K)=3600 μmKfλ2=0.403607

α=(0.1)0.000169+(0.8)(0.403607−0.000169)+(0.0)(1−0.403607)=0.323

15-94 The variation of absorptivity of a surface with wavelength is given. The surface receives solar radiation at a specified rate. The solar absorptivity of the surface and the rate of absorption of solar radiation are to be determined.



, m

0.3

1.2

0.8

0.1

0

Analysis  For solar radiation, T = 5800 K. The solar absorptivity of the surface is

λ1T=(0.3μm)(5800 K)=1740 μmK→fλ1=0.033454λ2T=(1.2μm)(5800 K)=6960 μmK→fλ2=0.805713

α=(0.1)0.033454+(0.8)(0.805713−0.033454)+(0.0)(1−0.805713)=0.621

The rate of absorption of solar radiation is determined from

Eabsorbed=αI=0.621(470 W/m2)=292 W/m2

15-95 The spectral transmissivity of a glass cover used in a solar collector is given. Solar radiation is incident on the collector. The solar flux incident on the absorber plate, the transmissivity of the glass cover for radiation emitted by the absorber plate, and the rate of heat transfer to the cooling water are to be determined.



, m

0.3

3

0.9

0

Analysis (a) For solar radiation, T = 5800 K. The average transmissivity of the surface can be determined from

τ(T)=τ1fλ1+τ2(fλ2−fλ1)+τ3(1−fλ2)

where fλ1 and fλ2 are blackbody radiation functions corresponding to λ1T and λ2T. These functions are determined from Table 15-2 to be

λ1T=(0.3μm)(5800 K)=1740 μmKfλ1=0.033454λ2T=(3μm)(5800 K)=17,400 μmKfλ2=0.978746

and

τ=(0.0)(0.033454)+(0.9)(0.978746−0.033454)+(0.0)(1−0.978746)=0.851

Since the absorber plate is black, all of the radiation transmitted through the glass cover will be absorbed by the absorber plate and therefore, the solar flux incident on the absorber plate is same as the radiation absorbed by the absorber plate:

Eabs. plate=τI=0.851(950 W/m2)=808.5 W/m2

(b) For radiation emitted by the absorber plate, we take T = 300 K, and calculate the transmissivity as follows:

λ1T=(0.3μm)(300 K)=90 μmKfλ1=0.0λ2T=(3μm)(300 K)=900 μmKfλ2=0.000169

τ=(0.0)(0.0)+(0.9)(0.000169−0.0)+(0.0)(1−0.000169)=0.00015

(c) The rate of heat transfer to the cooling water is the difference between the radiation absorbed by the absorber plate and the radiation emitted by the absorber plate, and it is determined from

Qwater=(τsolar−τroom)I=(0.851−0.00015)(950 W/m2)=808.3 W/m2

Air, Tf

Tw = 500 K

Thermocouple

Tth = 850 K

 = 0.6

15-96 The temperature of air in a duct is measured by a thermocouple. The radiation effect on the temperature measurement is to be quantified, and the actual air temperature is to be determined.

Assumptions The surfaces are opaque, diffuse, and gray.

Properties The emissivity of thermocouple is given to be =0.6.

Analysis The actual temperature of the air can be determined from

Tf=Tth+εthσ(Tth4−Tw4)h850 K+(0.6)(5.67×10−8 W/m2⋅K4)[(850 K)4−(500 K)4]60 W/m2⋅°C1111 K

15-97 Radiation heat transfer occurs between a tube-bank and a wall. The view factors, the net rate of radiation heat transfer, and the temperature of tube surface are to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 The tube wall thickness and convection from the outer surface are negligible.

Properties The emissivities of the wall and tube bank are given to be i = 0.8 and j = 0.9, respectively.

Analysis (a) We take the wall to be surface i and the tube bank to be surface j. The view factor from surface i to surface j is determined from

Fij=1−1−Ds20.5+Dstan−1sD2−10.51−1−1.5320.5+1.53tan−131.52−10.5=0.658

The view factor from surface j to surface i is determined from reciprocity relation. Taking s to be the width of the wall

AiFij=AjFjiFji=AiAjFij=sLπDLFij=sπDFij=3π(1.5)(0.658)=0.419

(b) The net rate of radiation heat transfer between the surfaces can be determined from

q=σTi4−Tj41−εiεi1Ai+1AiFij+1−εjεj1Aj=σTi4−Tj41−εiεi+1Fij+1−εjεjAiAj(5.67×10−8 W/m2⋅K4)1173 K4−333 K41−0.80.8+10.658+1−0.90.9(0.03 m)π(0.015 m)=57,900 W/m2

(c) Under steady conditions, the rate of radiation heat transfer from the wall to the tube surface is equal to the rate of convection heat transfer from the tube wall to the fluid. Denoting Tw to be the wall temperature,

qrad=qconvσTi4−Tw41−εiεi+1Fij+1−εjεjAiAj=hAj(Tw−Tj)(5.67×10−8 W/m2⋅K4)1173 K4−Tw41−0.80.8+10.658+1−0.90.9(0.03 m)π(0.015 m)=(2000 W/m2⋅K)π(0.015 m)0.03 mTw−(40+273 K)

Solving this equation by an equation solver such as EES, we obtain

Tw = 331.4 K = 58.4C

15-98E A sealed electronic box is placed in a vacuum chamber. The highest temperature at which the surrounding surfaces must be kept if this box is cooled by radiation alone is to be determined.

Tsurr

8 in

90 W

 = 0.95

Ts = 130F

12 in

12 in

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered. 4 Heat transfer from the bottom surface of the box is negligible.

Properties The emissivity of the outer surface of the box is  = 0.95.

Analysis The total surface area is

As=4×(8×112)+(1×1)=3.67 ft2

Then the temperature of the surrounding surfaces is determined to be

Qrad=εAsσ(Ts4−Tsurr4)(90×3.41214) Btu/h=(0.95)(3.67 ft2)(0.1714×10−8 Btu/h.ft2⋅R4)[(590 R)4−Tsurr4]Tsurr=514 R=54°F

15-99 A double-walled spherical tank is used to store iced water. The air space between the two walls is evacuated. The rate of heat transfer to the iced water and the amount of ice that melts a 24-h period are to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray.

Properties The emissivities of both surfaces are given to be  1 = 2 = 0.15.

Analysis (a) Assuming the conduction resistance s of the walls to be negligible, the rate of heat transfer to the iced water in the tank is determined to be

A1=πD12=π(2.01 m)2=12.69 m2

Q12=A1σ(T24−T14)1ε1+1−ε2ε2D1D22(12.69 m2)(5.67×10−8 W/m2⋅K4)[(20+273 K)4−(0+273 K)4]10.15+1−0.150.152.012.042107.4 W

(b) The amount of heat transfer during a 24-hour period is

Q=QΔt=(0.1074 kJ/s)(24×3600 s)=9279 kJ

The amount of ice that melts during this period then becomes

Q=mhifm=Qhif=9279 kJ333.7 kJ/kg=27.8 kg

15-100 Two concentric spheres which are maintained at uniform temperatures are separated by air at 1 atm pressure. The rate of heat transfer between the two spheres by natural convection and radiation is to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Air is an ideal gas with constant properties.

D2 = 25 cm

T2 = 275 K

2 = 0.75

D1 = 15 cm

T1 = 350 K

1 = 0.75

Lc =5 cm

AIR

1 atm

Properties The emissivities of the surfaces are given to be 1 = 2 = 0.75. The properties of air at 1 atm and the average temperature of (T1+T2)/2 = (350+275)/2 = 312.5 K = 39.5C are (Table A-22)

k=0.02658 W/m.°Cν=1.697×10−5 m2/sPr=0.7256β=1312.5 K=0.0032 K-1

Analysis (a) Noting that Di = D1 and Do = D2 , the characteristic length is

Lc=12(Do−Di)=12(0.25 m−0.15 m)=0.05 m

Then

Ra=gβ(T1−T2)Lc3ν2Pr=(9.81 m/s2)(0.003200 K-1)(350−275 K)(0.05 m)3(1.697×10−5 m2/s)2(0.7256)=7.415×105The effective thermal conductivity is

Fsph=Lc(DiDo)4(Di−75+Do−75)5=0.05 m(0.15 m)(0.25 m)4(0.15 m)-7/5+(0.25 m)-7/55=0.005900

keff=0.74kPr0.861+Pr14(FsphRa)140.74(0.02658 W/m.°C)0.72560.861+0.725614(0.00590)(7.415×105)140.1315 W/m.°C

Then the rate of heat transfer between the spheres becomes

Q=keffπDiDoLc(Ti−To)=(0.1315 W/m.°C)π(0.15 m)(0.25 m)(0.05 m)(350−275)K=23.2 W

(b) The rate of heat transfer by radiation is determined from

A1=πD12=π(0.15 m)2=0.0707 m2Q12=A1σ(T24−T14)1ε1+1−ε2ε2D1D22=(0.0707 m2)(5.67×10−8 W/m2⋅K4)[(350 K)4−(275 K)4]10.75+1−0.750.750.150.252=25.6 W

15-101 A solar collector is considered. The absorber plate and the glass cover are maintained at uniform temperatures, and are separated by air. The rate of heat loss from the absorber plate by natural convection and radiation is to be determined.

Solar radiation

 = 20

Insulation

Absorber plate

T1 = 80C

1 = 0.8

Glass cover,

T2 = 32C

2 = 0.9

1.5 m

L = 3 cm

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Air is an ideal gas with constant properties.

Properties The emissivities of surfaces are given to be  1 = 0.9 for glass and 2 = 0.8 for the absorber plate. The properties of air at 1 atm and the average temperature of (T1+T2)/2 = (80+32)/2 = 56C are (Table A-22)

k=0.02779 W/m.°Cν=1.857×10−5 m2/sPr=0.7212β=1Tf=1(56+273)K=0.003040 K-1

Analysis For θ=0°, we have horizontal rectangular enclosure. The characteristic length in this case is the distance between the two glasses Lc = L = 0.03 m Then,

Ra=gβ(T1−T2)L3ν2Pr=(9.81 m/s2)(0.00304 K-1)(80−32 K)(0.03 m)3(1.857×10−5 m2/s)2(0.7212)=8.083×104

As=H×W=(1.5 m)(3 m)=4.5 m2

Nu=1+1.441−1708Racosθ+1−1708(sin1.8θ)1.6Racosθ+(Racosθ)1318−1+1+1.441−1708(8.083×104)cos(20)+1−1708sin(1.8×20)1.6(8.083×104)cos(20)+(8.083×104)cos(20)1318−1+3.747	Q=kNuAsT1−T2L=(0.02779 W/m.°C)(3.747)(4.5 m2)(80−32)°C0.03 m=750 W

Neglecting the end effects, the rate of heat transfer by radiation is determined from

Qrad=Asσ(T14−T24)1ε1+1ε2−1=(4.5 m2)(5.67×10−8 W/m2⋅K4)[(80+273 K)4−(32+273 K)4]10.8+10.9−1=1289 W

Discussion The rates of heat loss by natural convection for the horizontal and vertical cases would be as follows (Note that the Ra number remains the same):

Horizontal:

Nu=1+1.441−1708Ra++Ra1318−1+=1+1.441−17088.083×104++(8.083×104)1318−1+=3.812

Q=kNuAsT1−T2L=(0.02779 W/m.°C)(3.812)(6 m2)(80−32)°C0.03 m=1017 W

Vertical:

Nu=0.42Ra14Pr0.012HL−0.3=0.42(8.083×104)14(0.7212)0.0122 m0.03 m−0.3=2.001

Q=kNuAsT1−T2L=(0.02779 W/m.°C)(2.001)(6 m2)(80−32)°C0.03 m=534 W

15-102E CD EES The circulating pump of a solar collector that consists of a horizontal tube and its glass cover fails. The equilibrium temperature of the tube is to be determined.

Assumptions 1 Steady operating conditions exist. 2 The tube and its cover are isothermal. 3 Air is an ideal gas. 4 The surfaces are opaque, diffuse, and gray for infrared radiation. 5 The glass cover is transparent to solar radiation.

D2 =5 in

Aluminum tube

D1 =2.5 in, T1

1 = 0.9

Air space

0.5 atm

Plastic cover,

2 = 0.9, T2

Water

T = 75F

Tsky = 60F

30 Btu/h.ft

Properties  The properties of air should be evaluated at the average temperature. But we do not know the exit temperature of the air in the duct, and thus we cannot determine the bulk fluid and glass cover temperatures at this point, and thus we cannot evaluate the average temperatures. Therefore, we will assume the glass temperature to be 85F, and use properties at an anticipated average temperature of (75+85)/2 =80°F (Table A-22E),

k=0.01481  Btu/h⋅ft⋅°Fν=1.697×10-4 ft2s

Pr=0.7290β=1Tave=1540 R

Analysis We have a horizontal cylindrical enclosure filled with air at 0.5 atm pressure. The problem involves heat transfer from the aluminum tube to the glass cover and from the outer surface of the glass cover to the surrounding ambient air. When steady operation is reached, these two heat transfer rates must equal the rate of heat gain. That is,

Qtube-glass=Qglass-ambient=Qsolar gain=30 Btu/h     (per foot of tube)

The heat transfer surface area of the glass cover is

Ao=Aglass=(πDoW)=π(512 ft)(1 ft)=1.309 ft2  (per foot of tube)

To determine the Rayleigh number, we need to know the surface temperature of the glass, which is not available. Therefore, solution will require a trial-and-error approach. Assuming the glass cover temperature to be 85°F, the Rayleigh number, the Nusselt number, the convection heat transfer coefficient, and the rate of natural convection heat transfer from the glass cover to the ambient air are determined to be

RaDo=gβ(To−T∞)Do3ν2Pr(32.2 ft/s2)[1(540 R)](85−75 R)(512 ft)3(1.697×10−4 ft2/s)2(0.7290)=1.092×106

Nu=0.6+0.387 RaD1/61+0.559/Pr9168272=0.6+0.387(1.092×106)161+0.5590.7290916827214.95

ho=kD0Nu=0.01481 Btu/h⋅ft⋅°F512 ft(14.95)=0.5315  Btu/h⋅ft2⋅°F

Qo,conv=hoAo(To−T∞)=(0.5315 Btu/h⋅ft2⋅°F)(1.309 ft2)(85−75)°F =6.96 Btu/h

Also,

Qo,rad=εoσAo(To4−Tsky4)(0.9)(0.1714×10−8 Btu/h⋅ft2⋅R4)(1.309 ft2)(545 R)4−(520 R)4       =30.5 Btu/h

Then the total rate of heat loss from the glass cover becomes

Qo,total=Qo,conv+Qo,rad=7.0+30.5=37.5  Btu/h

which is more than 30 Btu/h. Therefore, the assumed temperature of 85°F for the glass cover is high. Repeating the calculations with lower temperatures (including the evaluation of properties), the glass cover temperature corresponding to 30 Btu/h is determined to be 81.5°F.

The temperature of the aluminum tube is determined in a similar manner using the natural convection and radiation relations for two horizontal concentric cylinders. The characteristic length in this case is the distance between the two cylinders, which is

Lc=(Do−Di)2=(5−2.5)2=1.25 in=1.25/12 ft

Also,

Ai=Atube=(πDiW)=π(2.512 ft)(1 ft)=0.6545  ft2  (per foot of tube)

We start the calculations by assuming the tube temperature to be 118.5°F, and thus an average temperature of (81.5+118.5)/2 = 100F=560 R. Using properties at 100F,

RaL=gβ(Ti−To)L3ν2Pr=(32.2 ft/s2)[1(560 R)](118.5−81.5 R)(1.2512 ft)3[(1.809×10−4 ft2/s)0.5]2(0.726)=1.334×104

The effective thermal conductivity is

Fcyc=[ln(DoDi)]4Lc3(Di−35+Do−35)5=[ln(52.5)]4(1.25/12 ft)3[(2.512 ft)-3/5+(512 ft)-3/5]5=0.1466

keff=0.386kPr0.861+Pr14(FcycRaL)140.386(0.01529 Btu/h⋅ft⋅°F)0.7260.861+0.726(0.1466×1.334×104)140.03227 Btu/h⋅ft⋅°F

Then the rate of heat transfer between the cylinders by convection becomes

Qi, conv=2πkeffln(DoDi)(Ti−To)=2π(0.03227 Btu/h⋅ft⋅°F)ln(5/2.5)(118.5−81.5)°F=10.8 Btu/h

Also,

Qi,rad=σAi(Ti4−To4)1εi+1−εoεoDiDo(0.1714×10−8 Btu/h⋅ft2⋅R4)(0.6545 ft2)(578.5  R)4−(541.5 R)410.9+1−0.90.92.5 in5 in=25.0 Btu/h

Then the total rate of heat loss from the glass cover becomes

Qi,total=Qi,conv+Qi,rad=10.8+25.0=35.8 Btu/h

which is more than 30 Btu/h. Therefore, the assumed temperature of 118.5°F for the tube is high. By trying other values, the tube temperature corresponding to 30 Btu/h is determined to be 113°F. Therefore, the tube will reach an equilibrium temperature of 113°F when the pump fails.

15-103 A double-pane window consists of two sheets of glass separated by an air space. The rates of heat transfer through the window by natural convection and radiation are to be determined.

5C

15C

L = 3 cm

H = 2 m

Air

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Air is an ideal gas with constant specific heats. 4 Heat transfer through the window is one-dimensional and the edge effects are negligible.

Properties The emissivities of glass surfaces are given to be  1 = 2 = 0.9.  The properties of air at 0.3 atm and the average temperature of (T1+T2)/2 = (15+5)/2 = 10C are (Table A-22)

k=0.02439 W/m.°Cν=ν1atm0.3=1.426×10−5/0.3=4.753×10−5 m2/sPr=0.7336β=1(10+273) K=0.003534 K-1

Analysis The characteristic length in this case is the distance between the glasses, Lc=L=0.03 m

Ra=gβ(T1−T2)L3ν2Pr=(9.81 m/s2)(0.003534 K-1)(15−5)K(0.03 m)3(4.753×10−5 m2/s)2(0.7336)=3040

Nu=0.197Ra14HL−19=0.197(3040)1420.05−19=0.971

Note that heat transfer through the air space is less than that by pure conduction as a result of partial evacuation of the space. Then the rate of heat transfer through the air space becomes

As=(2 m)(5 m)=10 m2

Qconv=kNuAsT1−T2L=(0.02439 W/m.°C)(0.971)(10 m2)(15−5)°C0.03 m=78.9 W

The rate of heat transfer by radiation is determined from

Qrad=Asσ(T14−T24)1ε1+1ε2−1=(10 m2)(5.67×10−8 W/m2⋅K4)[15+273 K4−5+273 K4]10.9+10.9−1=421 W

Then the rate of total heat transfer becomes

Qtotal=Qconv+Qrad=79+421=500 W

Discussion Note that heat transfer through the window is mostly by radiation.

15-104 CD EES A simple solar collector is built by placing a clear plastic tube around a garden hose. The rate of heat loss from the water in the hose by natural convection and radiation is to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Air is an ideal gas with constant specific heats.

Properties The emissivities of surfaces are given to be  1 = 2 = 0.9.   The properties of air are at 1 atm and the film temperature of (Ts+T)/2 = (40+25)/2 = 32.5C are (Table A-22)

D2 =6 cm

Garden hose

D1 =2 cm, T1

1 = 0.9

Air space

Plastic cover,

2 = 0.9, T2 =40C

Water

T = 25C

Tsky = 15C

k=0.02607 W/m.°Cν=1.632×10−5 m2/sPr=0.7275β=1(32.5+273) K=0.003273 K-1

Analysis Under steady conditions, the heat transfer rate from the water in the hose equals to the rate of heat loss from the clear plastic tube to the surroundings by natural convection and radiation.  The characteristic length in this case is the diameter of the plastic tube, Lc=Dplastic=D2=0.06 m.

Ra=gβ(Ts−T∞)D23ν2Pr=(9.81 m/s2)(0.003273 K-1)(40−25)K(0.06 m)3(1.632×10−5 m2/s)2(0.7275)=2.842×105	Nu=0.6+0.387 RaD1/61+0.559/Pr9168272=0.6+0.387(2.842×105)161+0.5590.72419168272=10.30

h=kD2Nu=0.02607 W/m.°C0.06 m(10.30)=4.475 W/m2.°CAplastic=A2=πD2L=π(0.06 m)(1 m)=0.1885 m2

Then the rate of heat transfer from the outer surface by natural convection becomes

Qconv=hA2(Ts−T∞)=(4.475 W/m2.°C)(0.1885 m2)(40−25)°C=12.7 W

The rate of heat transfer by radiation from the outer surface is determined from

Qrad=εA2σ(Ts4−Tsky4)(0.90)(0.1885 m2)(5.67×10−8 W/m2⋅K4)[(40+273 K)4−(15+273 K)4]26.1 W

Finally,

Qtotal,loss=12.7+26.1=38.8 W

Discussion Note that heat transfer is mostly by radiation.

15-105 A solar collector consists of a horizontal copper tube enclosed in a concentric thin glass tube. The annular space between the copper and the glass tubes is filled with air at 1 atm. The rate of heat loss from the collector by natural convection and radiation is to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and  gray. 3 Air is an ideal gas with constant specific heats.

D2=12 cm

Copper tube

D1 =5 cm, T1 = 60C

1 = 0.85

Air space

Plastic cover,

T2 = 40C

2 = 0.9

Water

Properties The emissivities of surfaces are given to be  1 = 0.85 for the tube surface and 2 = 0.9 for glass cover. The properties of air at 1 atm and the average temperature of (T1+T2)/2 = (60+40)/2 = 50C are (Table A-22)

k=0.02735 W/m.°Cν=1.798×10−5 m2/sPr=0.7228β=1(50+273) K=0.003096 K-1

Analysis The characteristic length in this case is

Lc=12(D2−D1)=12(0.12 m-0.05 m)=0.07 m

Ra=gβ(T1−T2)Lc3ν2Pr=(9.81 m/s2)(0.003096 K-1)(60−40)K(0.035 m)3(1.798×10−5 m2/s)2(0.7228)=5.823×104

The effective thermal conductivity is

Fcyl=ln(DoDi)4Lc3(Di−35+Do−35)5=ln(0.120.05)4(0.035 m)3(0.05 m)-3/5+(0.12 m)-3/55=0.1678

keff=0.386kPr0.861+Pr14(FcylRa)140.386(0.02735 W/m.°C)0.72280.861+0.722814(0.1678)(5.823×104)14=0.08626 W/m.°C

Then the rate of heat transfer between the cylinders becomes

Qconv=2πkeffln(DoDi)(Ti−To)=2π(0.08626 W/m.°C)ln(0.120.05)(60−40)°C=12.4 W

The rate of heat transfer by radiation is determined from

Qrad=A1σ(T14−T24)1ε1+1−ε2ε2D1D2=π(0.05 m)(1 m)(5.67×10−8 W/m2⋅K4)[(60+273 K)4−(40+273 K)4]10.85+1−0.90.951219.7 W

Finally,

Qtotal,loss=12.4+19.7=32.1 W   (per m length)

15-106 A cylindrical furnace with specified top and bottom surface temperatures and specified heat transfer rate at the bottom surface is considered. The temperature of the side surface and the net rates of heat transfer between the top and the bottom surfaces, and between the bottom and the side surfaces are to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered.

Properties The emissivities of the top, bottom, and side surfaces are 0.70, 0.50, and 0.40, respectively.

T1 = 500 K

1 = 0.70

r1 = 0.6 m

T3 = ?

3 = 0.40

h = 1.2 m

T2 = 650 K

2 = 0.50

r2 = 0.6 m

Analysis We consider the top surface to be surface 1, the bottom surface to be surface 2, and the side surface to be surface 3. This system is a three-surface enclosure. The view factor from surface 1 to surface 2 is determined from

Lr=1.20.6=2F12=0.17	(Fig. 15-7)

The surface areas are

A1=A2=πD24=π(1.2 m)24=1.131 m2A3=πDL=π(1.2 m)(1.2 m)=4.524 m2

Then other view factors are determined to be

F12=F21=0.17

F11+F12+F13=10+0.17+F13=1F13=0.83  (summation rule),	F23=F13=0.83

A1F13=A3F31(1.131)(0.83)=(4.524)F31F31=0.21  (reciprocity rule), 	F32=F31=0.21

We now apply Eq. 15-35 to each surface

Surface 1:	        σT14=J1+1−ε1ε1F12(J1−J2)+F13(J1−J3)(5.67×10−8 W/m2.K4)(500 K)4=J1+1−0.700.700.17(J1−J2)+0.83(J1−J3)

Surface 2:	        σT24=J2+1−ε2ε2F21(J2−J1)+F23(J2−J3)(5.67×10−8 W/m2.K4)(500 K)4=J2+1−0.500.500.17(J2−J1)+0.83(J2−J3)

Surface 3:	        σT34=J3+1−ε3ε3F31(J3−J1)+F32(J3−J2)(5.67×10−8 W/m2.K4)T34=J3+1−0.400.400.21(J3−J1)+0.21(J3−J2)

We now apply Eq. 15-34 to surface 2

Q2=A2F21(J2−J1)+F23(J2−J3)=(1.131 m2)0.17(J2−J1)+0.83(J2−J3)

Solving the above four equations, we find

T3=631 K,   J1=4974 W/m2,   J2=8883 W/m2,    J3=8193 W/m2

The rate of heat transfer between the bottom and the top surface is

Q21=A2F21(J2−J1)=(1.131 m2)(0.17)(8883−4974)W/m2=751.6 W

The rate of heat transfer between the bottom and the side surface is

Q23=A2F23(J2−J3)=(1.131 m2)(0.83)(8883−8193)W/m2=648.0 W

Discussion The sum of these two heat transfer rates are 751.6 + 644 = 1395.6 W, which is practically equal to 1400 W heat supply rate from surface 2. This must be satisfied to maintain the surfaces at the specified temperatures under steady operation. Note that the difference is due to round-off error.

15-107 A cylindrical furnace with specified top and bottom surface temperatures and specified heat transfer rate at the bottom surface is considered. The emissivity of the top surface and the net rates of heat transfer between the top and the bottom surfaces, and between the bottom and the side surfaces are to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered.

T1 = 700 K

1 = ?

T2 = 950 K

2 = 0.90

T3 = 450 K

3 = 1

3 m

Properties The emissivity of the bottom surface is 0.90.

Analysis We consider the top surface to be surface 1, the base surface to be surface 2, and the side surface to be surface 3. This system is a three-surface enclosure. The view factor from the base to the top surface of the cube is from Fig. 15-5F12=0.2. The view factor from the base or the top to the side surfaces is determined by applying the summation rule to be

since the base surface is flat and thus F11=0. Other view factors are

F21=F12=0.20,          F23=F13=0.80,          F31=F32=0.20

We now apply Eq. 9-35 to each surface

Surface 1:	        σT14=J1+1−ε1ε1F12(J1−J2)+F13(J1−J3)(5.67×10−8 W/m2.K4)(700 K)4=J1+1−ε1ε10.20(J1−J2)+0.80(J1−J3)

Surface 2:	        σT24=J2+1−ε2ε2F21(J2−J1)+F23(J2−J3)(5.67×10−8 W/m2.K4)(950 K)4=J2+1−0.900.900.20(J2−J1)+0.80(J2−J3)

Surface 3:	        σT34=J3(5.67×10−8 W/m2.K4)(450 K)4=J3

We now apply Eq. 9-34 to surface 2

Q2=A2F21(J2−J1)+F23(J2−J3)=(9 m2)0.20(J2−J1)+0.80(J2−J3)

Solving the above four equations, we find

ε1=0.44,   J1=11,736 W/m2,   J2=41,985 W/m2,    J3=2325 W/m2

The rate of heat transfer between the bottom and the top surface is

A1=A2=(3 m)2=9 m2

Q21=A2F21(J2−J1)=(9 m2)(0.20)(41,985−11,736)W/m2=54.4 kW

The rate of heat transfer between the bottom and the side surface is

A3=4A1=4(9 m2)=36 m2

Q23=A2F23(J2−J3)=(9 m2)(0.8)(41,985−2325)W/m2=285.6  kW

Discussion The sum of these two heat transfer rates are 54.4 + 285.6 = 340 kW, which is equal to 340 kW heat supply rate from surface 2.

15-108 Radiation heat transfer occurs between two square parallel plates. The view factors, the rate of radiation heat transfer and the temperature of a third plate to be inserted are to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered.

Properties The emissivities of plate a, b, and c are given to be a = 0.8, b = 0.4, and c = 0.1, respectively.

Analysis (a) The view factor from surface a to surface b is determined as follows

a

b

L

a

b

A=aL=2040=0.5,  B=bL=6040=1.5Fab=12A(B+A)2+40.5−(B−A)2+40.5=12(0.5)(1.5+0.5)2+40.5−(1.5−0.5)2+40.5=0.592The view factor from surface b to surface a is determined from reciprocity relation:

Aa=(0.2 m)(0.2 m)=0.04 m2Ab=(0.6 m)(0.6 m)=0.36 m2AaFab=AbFba(0.04)(0.592)=(0.36)FbaFba=0.0658

(b) The net rate of radiation heat transfer between the surfaces can be determined from

Qab=σTa4−Tb41−εaAaεa+1AaFab+1−εbAbεb=(5.67×10−8 W/m2⋅K4)1073 K4−473 K41−0.8(0.04 m2)(0.8)+1(0.04 m2)(0.592)+1−0.4(0.36 m2)(0.4)=1374 W

(c) In this case we have

A=aL=0.2 m0.2 m=1,  C=cL=2.0 m0.2 m=10Fac=12A(C+A)2+40.5−(C−A)2+40.5=12(0.5)(10+0.5)2+40.5−(10−0.5)2+40.5=0.981

B=bL=0.6 m0.2 m=3,   C=cL=2.0 m0.2 m=10Fbc=12A(C+B)2+40.5−(C−B)2+40.512(3)(10+3)2+40.5−(10−3)2+40.5=0.979

AbFbc=AcFcb(0.36)(0.979)=(4.0)FcbFba=0.0881

An energy balance gives

Qac=QcbσTa4−Tc41−εaAaεa+1AaFac+1−εcAcεc=σTc4−Tb41−εcAcεc+1AcFcb+1−εbAbεb(1073 K)4−Tc41−0.8(0.04 m2)(0.8)+1(0.04 m2)(0.981)+1−0.1(4 m2)(0.1)=Tc4−(473 K)41−0.1(4 m2)(0.1)+1(4 m2)(0.0881)+1−0.4(0.36 m2)(0.4)

Solving the equation with an equation solver such as EES, we obtain  Tc = 754 K = 481C

15-109 Radiation heat transfer occurs between two concentric disks. The view factors and the net rate of radiation heat transfer for two cases are to be determined.

Assumptions 1 Steady operating conditions exist 2 The surfaces are opaque, diffuse, and gray. 3 Convection heat transfer is not considered.

Properties The emissivities of disk 1 and 2 are given to be a = 0.6 and b = 0.8, respectively.

Analysis (a) The view factor from surface 1 to surface 2 is determined using Fig. 15-7 as

2

1

L

Lr1=0.100.20=0.5,    r2L=0.100.10=1F12=0.19

Using reciprocity rule,

A1=π(0.2 m)2=0.1257 m2A2=π(0.1 m)2=0.0314 m2F21=A1A2F12=0.1257 m20.0314 m2(0.19)=0.76

(b) The net rate of radiation heat transfer between the surfaces can be determined from

Q=σT14−T241−ε1A1ε1+1A1F12+1−ε2A2ε2=(5.67×10−8 W/m2⋅K4)1073 K4−573 K41−0.6(0.1257 m2)(0.6)+1(0.1257 m2)(0.19)+1−0.8(0.0314 m2)(0.8)=1250 W

(c) When the space between the disks is completely surrounded by a refractory surface, the net rate of radiation heat transfer can be determined from

Q=A1σT14−T24A1+A2−2A1F12A2−A1F122+1ε1−1+A1A21ε2−1(0.1257 m2)(5.67×10−8 W/m2⋅K4)1073 K4−573 K40.1257+0.0314−2(0.1257)(0.19)0.0314−(0.1257)(0.19)2+10.6−1+0.12570.031410.8−1=1510 W

Discussion The rate of heat transfer in part (c) is 21 percent higher than that in part (b).

## 15-110 ….. 15-113 Design and Essay Problems

